UNCLASSIFIED

|  |  | REFERENCES | ISSUE | DATE |
| --- | --- | --- | --- | --- |

Streamlining Development Assurance µXAV Specification (Overarching Properties Oriented)

|  | SCOPE |
| --- | --- |

|  |  | Page : 1 / 46 |
| --- | --- | --- |

This document is the intellectual property of RESSAC members It may not be used, reproduced, modified or disclosed without its authorization. RESSAC members Proprietary Data.

signatures

|  | REFERENCES | APPROVERS |
| --- | --- | --- |
|  |  | Fonction 1 |
|  |  | Name 1 |
|  |  | Fonction 2 |
|  |  | Name 2 |
|  |  | Fonction 3 |
|  |  | Name 3 |
|  |  | Fonction 4 |
|  |  | Name 4 |
|  |  | Fonction 5 |
|  |  | Name 5 |

Table of contentS

1.	PREAMBULE	5

1.1	Document issues	5

1.2	Executive Summary	5

2.	Purpose of the document	6

3.	participants to the case study	7

4.	INTRODUCTION	8

4.1	Context	8

4.2	Overarching Properties	8

4.2.1	OP Intent	8

4.2.2	OP Necessity	9

4.2.3	OP Correctness	9

4.3	Collaborative Development	9

4.4	Intellectual Property Issues	9

4.5	Structure of the Document	9

5.	INDEX	12

6.	DESIRED BEHAVIOUR	14

6.1	Introduction	14

6.2	Air Vehicle Level	14

6.2.1	Operating Procedure	14

6.2.2	Expected Operating Performances	16

6.2.3	Foreseeable Operating Conditions	16

6.2.4	Validation scenarios	17

6.3	System Level	18

6.3.1	Electrical Propulsion System	19

6.3.2	Hydraulic Braking System	19

6.3.3	Mission Management System	20

6.3.4	Maintenance System	20

6.4	Equipment and Item Levels	21

7.	Architecting	23

7.1	Introduction	23

7.2	Design Rational	23

7.3	Air Vehicle	23

7.3.1	Man Machine Interface	23

7.3.2	Functional Architecture	26

7.3.3	Organic Architecture	27

7.3.4	Mapping of Functions on Systems	27

7.3.5	Mapping of Subfunctions on Systems	28

7.4	Communication Architecture	29

7.5	Electrical Propulsion System	30

7.5.1	Functional Architecture	30

7.5.2	Organic Architecture	30

7.6	Hydraulic Braking System	32

7.6.1	Functional Architecture	32

7.6.2	Organic Architecture	33

7.7	Mission Management System	34

7.7.1	Functional Architecture	34

7.7.2	Organic Architecture	35

7.8	Maintenance System	35

7.8.1	Functional Architecture	35

7.8.2	Organic Architecture	35

8.	Defined Intended Functions	36

8.1	Introduction	36

8.2	Air Vehicle Level	36

8.2.1	Payload Transport	36

8.2.2	Subfunctions of Payload Transport	41

8.2.3	Emergency Landing	46

8.2.4	Health Management	47

8.3	System Level	49

8.3.1	Introduction	49

8.3.2	EPS’ Functions	50

8.3.3	HBS’ Functions	61

8.3.4	MMS’ Functions	71

8.3.5	MS’ Functions	76

9.	IMPLEMENTATION	77

9.1	Electrical Propulsion System	77

9.2	Hydraulic Breaking System	78

9.3	Mission Management System	79

9.3.1	Software Implementation	79

9.3.2	Hardware Implementation	79

9.4	Maintenance System	80

10.	Appendix A : Overarching PROPERTIES	81

# PREAMBULE

## Document issues

| Date | Issue | Author(s) | Updating purpose |
| --- | --- | --- | --- |
| 22/08/2016 | 1 | E. Ledinot | Creation of a document containing only the specification of the case study and structured in a way facilitating the work on Overarching Properties |

## Executive Summary

This document is an intermediary step that concentrates all the specification traits of the µXAV in a way hopefully suited for working on the three Overarching Properties: Intent, Correctness, Necessity, as defined at the Westford FAA workshop.

Intent is addressed by the “Desired Behaviour” part.

Necessity is mainly addressed by the “Architecting” part.

Correctness is addressed by the “Defined Intended Functions” part and the “Implementation” part.

The document is not self-contained: the overall context of the µXAV case study (the safety analysis, the operational analysis and the functional analysis) are assumed to be known by the reader [Ref].

The purpose of the document is to provide the specification material needed to implement the use case.

# Purpose of the document

Specification of the drone µXAV at air vehicle level, system level and item level (software or hardware). Provides the material needed for system level modelling and software or hardware implementation.

More precisely the document addresses specification of the use case from air vehicle level up to allocation of the system requirements to the software or hardware items. Software and hardware high level requirements (HLRs), and LLRs, are not contained in this document.

This document is associated with three other documents:

- µXAV Safety Analysis and Assessment: contains the safety process part of the case study, that is outside of the scope of FAA’s initiative but key for the Correctness and Intent Overarching Properties,

- µXAV Baseline Development Assurance: contains the outputs of some development assurance activities related to µXAV development, that are conformant to ARP 4754, DO 178, DO 254.

- µXAV Steamlined Development Assurance: contains the outputs of some development assurance activities related to µXAV development that are conformant to the “Overarching Properties” approach (formely named Meta-Objectives or TIAM)

This document, like the associated ones, the development artefacts and the assurance artefacts, are available be a GitHub collaborative workspace (https://github.com/AdaCore/RESSAC_Use_Case), for joint work by industrials, academics and Authorities.

# participants to the case study

| Guy-André Berthon | THALES |
| --- | --- |
| James Bezémat | SAFRAN |
| Duncan Brown | ROLLS ROYCE |
| Jean Louis Camus | ANSYS/Esterel Technologies |
| Nicolas Chevillard | DGA |
| Cyrille Comar | AdaCore |
| Olivier Constant | IRT St Exupery |
| Miguel De Almeda | APSYS-Airbus |
| Alexandre Jordan | APSYS-Airbus |
| Emmanuel Ledinot | DASSAULT AVIATION |
| Anne Perrine Porte | ZODIAC Aerospace |
| Frederic Pothon | ACG Solutions |
| Jose Ruiz | AdaCore |
| Virginie Wiels | ONERA |

# INTRODUCTION

## Context

The case study is meant to add a bottom-up approach to the top-down approach initiated by Mike Dewalt, CSO of FAA for airborne software certification until April 2016. It is intended to experiment with the TIAM development assurance method (Technology Independent Assurance Method). Accordingly, it will support attempts to get insights on acceptance criteria for the formerly so-called Meta Objectives, now called Overarching Properties (see Westford FAA workshop, 2016, July 13-16th).

“Overarching” has a twofold interpretation: hierarchical decomposition and development assurance perimeter. The three development assurance “overarching properties”, OPs, encompass:

- All the development assurance goals of the ARP 4754, DO 178, DO 254,

- All the development activities at air vehicle, system and item levels.

There are three Overarching Properties: Intent, Correctness and Necessity (formerly and respectively MO3, MO1, MO2).

## Overarching Properties

The OPs are assumed to be applied at each development step of the development process, when identified input and output artefacts are available. They are the high level goals, the intent core of development assurance to be applied to this development step.

We recall the formulation of the OPs, not their additional Definitions / Prerequisites / Constraints / Assumptions sections. See Appendix A for a comprehensive version of the Overarching Properties.

The content of the Prerequisites / Constraints / Assumptions sections is key for defining OP satisfiability criteria since they specify the constraints over the arguments that can be used to demonstrate that an OP on the step’s input & output artefacts is satisfied.

The three Ops are not ranked or ordered. They have to be simultaneously satisfied at the end of the development process. It is still debated whether or not DAL modulation (at low DALs) may modify the set of OPs to be met.

### OP Intent

The defined intended functions are correct and complete with respect to the desired system behavior

This goal addresses the development assurance activities in charge of checking the quality of requirement capture and validation (formely MO3).

### OP Necessity

All of the implementation is either required by the defined intended function or is without unacceptable safety impact.

This OP enforces minimization of potential sources on harmful non intended functions. It is less stringent than the (presumed)-dead code elimination policy of DO-178. It is the only OP that might be discarded at lower DALs.

### OP Correctness

The implementation is correct with respect to its defined intended functions, under foreseeable operating conditions.

It is interesting to note that “correct” is not defined in the Definition section of this OP, in spite of the central role played by this notion in the OP.

‘Correctness’ is defined in formal methods. There are various ways dependent on the nature of the artefacts. The document tries to help the WG having an effective approach to Correctness on the use case by distinguishing two kinds of specification traits:

- Declarative: synonyms are predicative, assertional, relational, constraints, properties. They state yes/no acceptance criteria over the “values” elaborated by the dynamics of the target of evaluation,

- Constructive: synonyms are behavioural, algorithmic (informational part only). Often, even at high level of abstraction, the needs are expressed as solutions instead of needs, as how things should happen, i.e a behaviour. In some cases, especially in control and signal processing, there is no other means of specifying what has to be done than giving how to do it (a mathematical function or representation).

## Collaborative Development

Joint work of the RESSAC participants is expected at two levels:

- Specification and implementation of the µXAV case study: Dassault Aviation has provided the initial framework, open to be supplemented and evolved as needed by welcomed contributors, especially SAFRAN on the AEH parts,

- Testing of the streamlined development assurance approach on the µXAV development artefacts.

## Intellectual Property Issues

DASSAULT AVIATION, SAFRAN and any other contributor to the use case contribute IP-free material. Similarly, IRT St Exupery claims no IP on the case study.

## Structure of the Document

The document is structured to help isolate the specification material relevant for a given OP.

The vocabulary used in the specification is identical to that used in OPs’ formulation.

The section named “Desired Behaviour” gathers the customer needs and the tier level needs in the refinement process from air vehicle level to item level. This material is intended to support validity issues (baseline assurance), and work on the acceptance criteria for OP ‘Intent’.

The section named “Architecting” describes the AV level and system level architectures. It also describes the rational that led to these architectural choices. It is intended to provide most of the material needed to work on the acceptance criteria for OP ‘Necessity. The two remaining sections contain also some traits relevant for Necessity.

The two remaining parts of the document address OP Correctness. Correctness is regarded as ‘conformance’ of an output artefact with respect to an input artefact at a given development step. What does ‘conformance’ mean?

The document tries as far as possible to separate the declarative and constructive specifications to distinguish three types of correctness, or Conformance the two words are regarded as synonymous in this document:

| Input Artefact | Output Artefact | Correctness Type |
| --- | --- | --- |
| Set of constraints and behavioural properties Declarative | Enriched Set of constraints and behavioural properties Declarative | The set of the solutions to the output set of constraints must be included into the set of solutions to the input set of constraints (refinement narrows the solution space) Set Inclusion |
| Set of properties Declarative | Definition (model or code) of a behaviour expected to meet the properties Constructive | All the relevant behaviours must meet all the properties Satisfiability Relation |
| Definition (model or code) of a behaviour Constructive | Enriched or translated behaviour Constructive | Any behaviour of the output artefact, once appropriately “traced back” to the corresponding input behaviour, is a possible behaviour of the input artefact Behavioural Inclusion |

The section named “Defined Intended Functions” (DIF) contains the functional specification form air vehicle level to equipment level. Allocation to software or AEH items is trivial (one to one). A DIF may be purely declarative, purely constructive, or a declarative-constructive mix. The constructive parts are algorithms defined at mathematical level (e.g transfer functions, state representations), or in any behavioural modelling language (model-based development).

The section named “Implementation” contains all the use case definition material related to the design decisions made to meet the DIF and realize the µXAV product. Derived declarative or constructive specifications are possible in this section, especially when the algorithmic choices are left to the implementer (e.g communication protocols). Model-based development is mainly expected at DIF level but may also occur at implementation level depending on the frontier between the specifier and the implementer.

In the µXAV case study, DIF is declarative and constructive at system level. The behavioural part is devoted to functional and performance early verification by the drone manufacturer and the system suppliers. At equipment and item levels it will depend on the various contributors to the use case.

The use case will illustrate several tier-tier technical agreements and responsibility frontiers as it is a key condition on development assurance policies.

# INDEX

| Acronym | Definition |
| --- | --- |
| ATC | Air Traffic Control |
| AV | Air Vehicle |
| AWC | Adverse Wind Conditions |
| BAT | Battery |
| BITE | Built-In Test Equipment |
| CCA | Common Cause Analysis |
| CL | Confidence Level |
| CMA | Common Mode Analysis |
| CPS | Cyber Physical System |
| ECU | Electronic Control Unit |
| EPS | Electrical Propulsion System |
| FB | Failure Bit |
| FC | Fuel Cell |
| FCs | Failure Conditions |
| FCS | Flight Control System |
| F_BC | Function Braking Control |
| F_CM | Function Communication Management |
| F_EL | Function Emergency Landing |
| F_EM | Function Energy Management |
| F_MC | Function Mission Control |
| F_MM | Function Mission Management |
| F_PC | Function Propulsion Control |
| FDIR | Fault Detection Isolation Recovery |
| FWC | Favourable Wind Conditions |
| GNC | Guidance Navigation Control |
| HBS | Hydraulic Braking System |
| HVCC | High Voltage Continuous Current |
| IS | International System (of physical units) |
| ISO OSI | International Standard Organisation Open Systems Interconnection |
| kt | Knot |
| LED | Light Electroluminescent Diode |
| MMI | Man Machine Interface |
| MS | Maintenance System |
| MMS | Mission Management System |
| MTOW | Maximal Take-Off Weight |
| MTTR | Mean Time To Repair |
| n.m | nautic miles |
| PDI | Parameter Data Item |
| PEPS | Primary Electrical Power Source |
| PI | Proportional Integral |
| SEPS | Secondary Electrical Power Source |
| SIC | Standard Icing Conditions |
| SWC | Standard Wind Condition |
| UAV | Unmanned Air Vehicle |

# DESIRED BEHAVIOUR

## Introduction

‘Desired behaviour’ is defined as “the system needs and constraints expressed by the stakeholders”.

‘Defined intended functions’ is defined as “The record of” followed by a verbatim copy of Desired Behaviour ‘s definition.

What is the difference between ‘expressed’ and ‘recorded expressed’?

In this document the following approach was adopted:

- Desired Behaviour is the “loose” specification of the use case, as it would supposedly be expressed by a user (or client recursively at lower tiers),

- Defined Intention Function is the “tight” specification, precise enough for implementation (supplier’s view at lower tiers). However DIF is still a specification, possibly constructive, but not an implementation.

Both specification contents are informal (‘formal’ as defined by DO 333), but hopefully significantly different.

## Air Vehicle Level

µXAV Air Vehicle is dedicated to 7/7-H24 autonomous or remotely controled cargo transport. The payloads are of small size (< 10kg).

Flight must be ensured for a large domain of weather conditions, whose definition is simplistically restricted to some wind vector distribution.

Availability, reliability, robustness, energy efficiency are the key performance design drivers of its embedded systems.

### Operating Procedure

The payload bay of µXAV is equipped with a Control Panel (CP) featuring (see figure in Architecting section):

- A USB plug for mission parameter upload (in case of no data-link or A mode),

- An ON/OFF general power-on button (two stable positions, lighted when ON),

- An OPEN/CLOSE switch (two stable positions) associated with a two-Rotactor mechanical ‘keyboard’. The button is set by the operator in charge of the payload. He has to enter the mass of the payload by means of the two mechanical cranked Rotactors (0-9, kg),

- An A/RP switch that configures the drone either for Autonomous flight, or for RP mode where communications with the Ground Station (GS) are active.

- A START push button (one unstable position) that launches the mission in A mode.

- A row of four mission indicators (lighted when the label is true), whose status is transmitted to the ground station:

  - READY: the various systems initialized successfully, the mission can be started,

  - COMPLETE: end of the mission in normal situation (landing at the right place in satisfactory conditions),

  - ABORTED: the mission was stopped before landing at the waypoint where the payload had to be delivered. The indicator is lighted after soft and hard landing. Soft landing occurs when enough energy and resources are available and control succeeds (anticipated and mastered situation), hard landing occurs when there is shortage of energy or too many failures. Control operates in a

  - CANCELED: no take-off because of any failure or energy problem

- Two displays: primary source capacity (%), secondary source capacity (%)

- An array of 20 red LEDs signaling the failure modes diagnosed by the maintenance system (see the figure in Architecting section).

µXAV can be operated without any ground station, in A mode (fully autonomous), using the control panel.

It can also be operated remotely by a ground station operator, e.g in case of convey (no payload), once payload in/out transfer is completed by someone besides the drone, if any.

Any mix of the two modes is possible: first autonomous, then a ground station operator takes over and modifies the mission modes and parameters (anything else impossible), or the ground station presets and initiates the mission, then lets it to complete in fully autonomous mode.

The missions are assumed to be defined by a so-called ‘flight plan’, a departure time, a distance, an expected arrival time, a maximum speed, and a flight level. Later versions will include waypoints.

The safety objectives span from medium criticality for cargo transport, to future high criticality for a planned piloted version.

In Remotely Piloted mode µXAV two-way communicates with its ground station:

- AV to GS:

  - the state vector, i.e speed, altitude, position, and the 0/1 status of all failure modes handled by the Health Monitoring functions.

  - after mission completion, a message with the remaining electrical capacities of the two sources.

- GS to AV:

  - The mode (RP, A) and navigation options (energy efficiency, time efficiency, auto),

  - The navigation parameters: distance, cruise altitude and speed,

  - Possible updates of the mode, parameters or option during the course of the mission.

A mission scenario consists in:

- Preparation of the AV: loading of the payload, uploading of the mission parameters by data-link or by mean of a USB stick plugged in the control panel, “refuelling” the primary and secondary electrical sources,

- Start of the mission by means of the data-link, or by pushing the START button on the control panel. It triggers power-on and initialization of the systems until status ‘Mission Ready’ or ‘Mission Cancelled” is set,

- Take-off, cruise and landing (normal or abnormal),

- Mission termination: remaining capacities displayed on the control panel and sent to the ground station. Power-off.

### Expected Operating Performances

Mission setup duration should be less than 15 mn when energy “refuelling” is needed, and less than 1 mn otherwise.

The drone flight conditions should meet the ATC regulatory requirements (flight levels, climb rate etc.)

Range at Maximum Take-Off Weight (MTOW) should be greater than 100 n.m in good weather conditions, and greater than 50 n.m in bad ones (but still within µXAV’s operation conditions).

Maximal cruise speed in favourable wind conditions should be at least 75kt for payload mass lower than 10kg.

When weather conditions are worse than that of operation conditions, a mission abortion logic followed by emergency landing should prevent the loss of the drone and ground damage or casualties. The target failure rates are 10-4 and 10-3 per mission, respectively for catastrophic and hazardous.

Downtime (energy refuelling and maintenance) of the drone should be less than 3 hours per 10.000 flight hours.

### Foreseeable Operating Conditions

The ‘foreseeable operating conditions’ encompass the external and internal disturbances that may occur during a mission. They also include any operator error, in the ground station or side of the drone. For instance:

- Weather conditions (wind and icing) may stay within the specified normal domain (good + bad), or leave this domain.

- Mass of the payload may be entered into the systems in three different ways: rolling the Rotactors on the control panel, from the ground station, or from the USB key. Incoherence between these values may occur and has to be addressed,

- A misleading payload mass value may occur even in case of threefold coherency. There is no payload weight sensor aboard the AV. The actual mass has to be estimated by the mission management software,

- In RP mode, arbitrary interleaving of ground station parameter updates and onboard system decisions may occur,

- Breakdowns may occur at any time on electrical, hydraulic, and digital hardware items. It is assumed that there is no failure of the AV’s mechanical body and of the data bus’ cable. Loss of digital communications (internal and/or external) are however possible in case of emitter or receiver failure.

- Cascading effects (e.g electrical -> hydraulic -> mechanical) or thermal effects may lead to many combination of item failures. It may also lead to latent failures before take-off.

- Cyber-attacks are out of the scope of the use case,

### Validation scenarios

The scenarii are indeed scenario patterns: many variants can be imagined depending on the external and internal events coming to play, and their relative ordering.

#### Scenario 1: Nominal

- Payload loading, switch in OPENED position at CP,

- Power ON and initialisation of the systems, initialisation completes in a few minutes, READY lights turns on,

- Initialisation of the mission on the AV by means of the USB stick, left in the slot. Mission parameters:

  - A mode (fully Autonomous),

  - Energy efficiency option

  - Declared payload mass: 5kg,

  - Distance: 60 n.m,

  - Flight level: 1000ft

  - Speed: 50 kt

- Payload bay closed, switch on CLOSED position,

- Take-off, climb, and beginning of cruise at preset altitude and speed,

- Take-over of the ground station to modify the cruise parameters as wind seems stronger than forecast.

- The ground station operator decreases cruise speed by 10kt,

- Continued cruise at new speed,

- Descent and AV landing at preset distance,

- The operator moves the switch to OPENED and off boards the payload. The systems are still powered for diagnosis and prognosis, and immediate take-off,

- Everything OK on the control panel (no energy “refuelling” needed for either of the two sources),

- Power off.

#### Scenario 2: Bad weather conditions no failures

- Same as scenario 1, with a shorter mission range, until mid of cruise phase,

- Wind gusts challenge A mode’s control laws. The AV makes transient forays in the hazardous flight domain,

- The hazardous transients are more and more frequent but escape catastrophic conditions. No take over by the GS is possible, communication latency is incompatible with short term control (remote piloting),

- The energy capacity is compatible with completion of the mission in spite of the electrical/hydraulic intensive controls to compensate the wind disturbances,

- Possible variant: add icing conditions up to trigger of emergency landing.

#### Scenario 3: Non critical breakdowns in good weather conditions

- Same as scenario 1 with addition of some resource failures, tolerated by recovery mechanisms:

  - Loss of digital communication links: external, internal

  - Loss of one electrical motor,

  - Loss of one hydraulic pump

#### Scenario 4: Critical breakdowns in bad weather conditions

Cumulative version of scenario 2 and scenario 3: external disturbances + internal disturbances, leading or not to mission completion. When the mission is aborted there are two possible variants:

- Abortion is decided in conditions compatible with fully controlled landing (soft emergency landing), i.e propulsion is still active. There is enough electrical energy to counteract the wind impulses in the descent and touch-down phases,

- Abortion is decided in conditions where there is no longer propulsion. It’s going to be a braking only descent and touch-down (hard landing).

## System Level

From the drone operator point of view there are few needs expressed at system level, and none at lower levels. At system level the desired behaviours are expressed by the drone manufacturer or by the system suppliers.

There are four systems embedded in µXAV:

- The Electrical Propulsion System (EPS), which converts chemical energy into mechanical energy of the AV’s body. EPS can only give energy (potential or kinetic) to the AV’s body. It can’t slow it down.

- The Hydraulic Braking Systems (HBS), in charge of decreasing the AV’s energy,

- The Mission Management System (MMS),

- The Maintenance System (MS).

Note: The first version of µXAV has no real flight dynamics and flight controls, but a simpler mechanical evolution law, for which hydraulic actuation makes sense. Future versions might be closer to actual flight but with electric only actuation.

### Electrical Propulsion System

There are two electrical energy sources, a primary one and a secondary one. The actual technology of these sources is not defined yet (to be chosen between batteries, super capacitors, and possibly small hydrogen fuel cells).

The primary source is slow to respond to load demands, with relatively low power peaks, but good energy capacity matching the range and speed objectives. The secondary source has a dual profile: quick response time, high power peaks but limited energy capacity. The secondary source is used for fast power up of the systems at init, for power intensive transients (e.g counteracting wind gusts), and in case of primary source exhaustion (soft emergency landing).

Short flights powered only with the secondary source should be possible, in degraded safety conditions (no redundancy). The secondary source is assumed to be a fail-free component, contrary to the primary source that may be a complex system per se (fuel cell option) with several failure modes.

The desired behaviour of EPS is to accurately and timely (20 Hz) convert the mechanical commands from the AV flight controller (MMS) into voltage commands on the electrical motors. EPS is in slave mode with respect to MMS, it never operates autonomously, contrary to HBS in case of emergency landing.

EPS monitors the primary and secondary energy consumptions, computes the remaining capacities and sends the datas to MMS (1Hz) which update mission viability analysis accordingly.

EPS monitors the degradations and failures of its resources (motors, switches, ECUs), and ensures a fail-safe policy, as far as possible.

### Hydraulic Braking System

HBS is in charge of degrading AV’s mechanical energy into thermal energy, either to lower altitude or speed or both of them. In this first version of the use case the thermal energy produced is not taken into consideration. But thermal coupling in case of excessive braking may cause multiple failures on electronic components to be addressed by CCA of the safety process. As there is no air conditioning systems in µXAV (like in many drones), thermal coupling is an interesting source of common cause failures and cascading failures, to be considered in later versions of the use case.

For lasting actions on brakes, or fast periodic ones, HBS needs electrical power to generate hydraulic power. Hydraulic pressure is supplied by two electrical pumps. An hydraulic accumulator provides limited pressure capacity for autonomous functioning of HBS, used only in case of emergency landing with complete loss of electrical power.

The desired behaviour of HBS is analogous to that of EPS:

- accurate and timely (20 Hz) conversion of mechanical commands into pressure commands,

- fail operational and then fail safe policy for health monitoring and reconfiguration.

### Mission Management System

MMS features the role of the software intensive systems in true avionics systems. In spite of the fact that µXAV has no large applications like Flight Management Systems (FMS) or complex Flight Control Laws (FCL), MMS is essentially a piece of software embedded in a dual channel computer. For the first version of the case study it is a mono-task mono-rate non interruptible software.

It performs mission control and EPS HBS supervision, depending on:

- mission preparation,

- possible in-flight updates of the navigation options by the ground station

- effects of weather conditions on the AV’s mission progress

- occurrence of failures

MMS receives (with redundancy) the AV’s sensors (pitch, pitch rate and roll) and ensures the mechanical control of the airframe. It ensures the role usually devoted to Guidance Navigation and Control (GNC) software in aircraft or rotorcraft. It is in charge of ensuring the safety properties of the AV’s dynamics (safety flight envelope). It receives kinematic measurements and emits mechanical control commands to EPS and HBS (torques). It operates exclusively in the mechanical energy domain.

MMS monitors energy, resource availability, and mission progress. Depending on these three sets of states it decides if the mission can proceed or has to be aborted, with soft or hard landing.

Hard landing is the Emergency Landing function (synonymous in the document). It is supported by HBS only (see Architecting). It happens when electrical energy is no longer available. Soft landing (to be distinguished from normal landing) is mission abortion with enough electrical energy to appropriately control the descent and the touchdown in the given external conditions.

MMS is also in charge of managing the digital internal and external communications. Internal communications are partly analog and partly digital (data bus). MMS software manages the bus. The bus cable is assumed to be a failure-free component.

MMS supports for the Health Management Systems the diagnosis and prognosis algorithms that operate at multi-system level. EPS and HBS have their health monitoring functions in charge of their own perimeter (See Architecting).

### Maintenance System

MS has no specific resources in this first version of the case study, and yet is posed as a system. Why?

First the maintenance system is usually an important and large system with proper resources in common aircraft and rotorcraft. Second in upgraded versions of µXAV a mass data recorder specific to MS is likely to be added for prognosis functions.

In this first version the sensors and actuators are assumed to be ideal (no bias, no lag, no noise etc.). But they may fail stop. So health monitoring boils down to Boolean status monitoring, Boolean logic and reporting.

In later versions these idealness assumptions will be progressively removed (first sensors, then actuators) to provide the use case with representative robustness issues, and their associated development assurance aspects. Robustness is a critical topic of development assurance, both at system and software levels, to be addressed by the use case at a significant level.

So the desired behaviour of MS is accurate detection, isolation and reporting of component failures or degradations. Accuracy means no false positives and no false negatives. MS operates in a stratified way: the MS module located in EPS and HBS detects, estimates and isolates the faults or degradations of its system and then reports its findings to the MS module of MMS, which in turn aggregates and consolidates when possible (See Architecting).

## Equipment and Item Levels

In this specification there is no desired behaviour proposed at equipment and items levels. This is consistent with the operator’s (client), drone manufacturer’s (tier 1) or system suppliers’ (tier 2) point of view adopted in this document. The specifications ends with the system requirements allocated to the software and AEH items: end of ARP 4754, beginning of DO-178 or DO-254.

So how do we proceed to split the system-level DIFs into system requirements allocated to software or hardware items?

The situation is the following:

- EPS and HBS have a unique Electronic Controller Unit (ECU). They are duplicated for fault tolerance reasons, but functionally speaking in both systems there is a unique “smart unit”. All other system components are physical devices: pumps, pipes, motors, inductances etc. Design and assurance of these components are outside of the scope the use case. They have to be modelled as environment models for closed loop simulation of the interacting “smart units”, but no functional behaviour is mapped on them. Because of this uniqueness, allocation of system level functionality (desired and defined) to the lower levels is trivial: all on the unique ECU.

- For MMS and MS the situation is even simpler: they are pure software systems, hosted in the same dual channel computer, or nearly so. To be precise:

  - MMS software is centralized in a single computer. All MMS functions (desired and defined) are mapped on the mission computer, and then on its unique software item (mono-task assumption).

  - MS functions are mapped on three control units (EPS’ controller, HBS’ controller and MMS’ mission computer) instead of one. But the mapping of MS functions to the AEH items (EPS’, HBS’, MMS’) and to the mission software item is straightforward: the subsidiarity principle is applied on the stratified functionalities, as explained previously (see also Architecting).

However it happens that some functional aspects at equipment or item levels (tier 3 and possibly tier 4) are addressed in the system level Defined Intended Functions and Implementation sections.

# Architecting

## Introduction

This section has two purposes:

- It concentrates all the architecture diagrams, synoptics and interface dictionaries for quick access to µXAV reference data and to an ‘helicopter view’ of the use case,

- It strives to facilitate work on OP Necessity by systematically outlining why the components of the architectures where necessitated by the Desired Behaviour and the Defined Intended Functions.

## Design Rational

Why three functions and four systems at AV level?

If µXAV had only one function in it would be the payload transportation function. In case of severe adverse conditions this function can be interrupted while in progress. An emergency function (emergency landing) takes over and is designed to operate at far lower energy consumption rate than ‘Payload Transportation’’s.

In all avionic systems (aircraft or rotorcraft) the maintenance system is distinct from the vehicle management system and from the mission management system. Hence the distinction in the use case as well. Moreover an extensive part of the maintenance functions operate on ground contrary to the two previous ones which are flight functions.

In addition the DALs of maintenance functions are often significantly lower than that of flight functions. Hence the three AV level functions.

Why four systems?

The use case was designed to be as simple as possible, but with as many seeds of real-life complexity as possible. Multi-physics with physical + digital interactions between the energy domains is one them. Hence three energy domains and two coupled systems: EPS and HBS in collaborative control of the same mechanical body.

To this physical part had to be added a mission management part, and a maintenance part to meet the availability objectives (by means of predictive maintenance). Hence the four systems.

## Air Vehicle

### Man Machine Interface

Two devices are involved:

- A control panel (CP), located on the downside of the drone near the payload bay

- A ground station, whose definition is limited to the message flows with the drone

#### Control Panel

Figure 2: The Control Panel Front-end

#### Ground Station

Upstream

The flow is event driven, contrary to the downstream one which has only periodic messages (1 Hz).

It is assumed that the communication protocol and the data-link technology ensure no message loss and at most 100ms latency.

| Message Name | Content | Subscriber(s) |
| --- | --- | --- |
| GS_MissionSetUp | Structure Type: [Distance:float; Speed:float; Altitude:float] | MMS.Controller[1,2] |
| GS_NavigationMode | Enum Type: {‘RP’,’A’} | MMS.Controller[1,2] |
| GS_NavigationOption | Enum Type: {‘SPEED’,’ENERGY’} | MMS.Controller[1,2] |
| GS_GO | None | MMS.Controller[1,2] |
| GS_EmergencyLanding | None | MMS.Controller[1,2] |

Message GS_GO starts the mission from the GS in RP mode.

Message GS_EmergencyLanding is effective only in RP mode

In RP mode the START button of the control panel is inactive.

Downstream

A first set of messages downloads the states of the CP, for ground-based mission supervision.

A second set of messages is provisioned to broadcast the sensor values used by the MMS computer, in case of needed in-flight or post-flight technical investigations.

| Message Name | Content | Subscriber(s) |
| --- | --- | --- |
| AV_CP_Switches | Structure Type: [Power: Boolean; Mode: {‘RP’,’A’}; Bay:{‘OPEN’,’CLOSED’}; START:Boolean; Rotactor1:Integer; Rotactor2:Integer;} ] | GS |
| AV_CP_Displays | Structure Type: [READY:Boolean; CANCELLED:Boolean; COMPLETE:Boolean; ABORTED:Boolean; FC_CAPACITY:Integer; BAT_CAPACITY:Integer ] | GS |
| AV_CP_LEDs | Bitfield 3 bytes for 2x10 logical bits Padding TBDL | GS |
| AV_Sensors_Mechanical | Provision | GS |
| AV_Sensors_Electrical | Provision | GS |
| AV_Sensors_Hydraulic | Provision | GS |
| AV_MissionCompleted | Structure Type: [PRIMARY_SOURCE:Integer; SECONDARY_SOURCE:Integer; ] | GS |

### Functional Architecture

The perimeter of the case study includes all the embedded systems of µXAV. It excludes the ground station but includes its communications with the drone.

So at AV level there are three groups of exchange with the external environment:

- The exchanges through the man machine interface (CP),

- The exchanges with the GS,

- The physical interactions of EPS and HBS with the AV’s airframe that contains the four embedded systems, by means of the mechanical sensors and actuators:

  - Sensors: pitch (p), pitch rate (pdot), roll (q). The roll rate is not measured.

  - Actuators: in this first version (no flight mechanics), action of thrust and air brakes are transposed into to two torques.

Figure 3: Functions and subfunctions at Air Vehicle Level

### Organic Architecture

Figure 4: The four embedded systems of µXAV

As previously explained, the maintenance system has no proper resources (until upgraded with a flight data recorder) and is distributed over the other systems.

### Mapping of Functions on Systems

Figure 5: Mapping of the AV level functions on the embedded systems

Payload Transport needs the three main systems: the two vehicle management systems (EPS and HBS) and the avionic system in charge of mission management (MMS).

The function of so to speak “best effort controlled crash”, alias F_Emergency_Landing (F_EL), is mapped exclusively on HBS because it involves only locally controlled braking.

F_EL may use hydraulic pressure supplied by the electrical pumps if there is still some electrical energy. But this possible hydraulic pressure from electrical origin accounts only for control margins. The emergency braking control law does not assume availability of any electrical energy (save that needed for HBS computations and considered as negligible), it only assumes full capacity of the hydraulic accumulator.

From the informational point of view, F_EL uses no data from MMS or EPS. It has direct access to the mechanical sensors by means of its own analog links, that remain operative even in case of loss of the primary electrical source, and low capacity of the secondary one.

So F_EL is mapped exclusively on HBS in spite of some possible energy giving influence of EPS on the behaviour of the function.

### Mapping of Subfunctions on Systems

Figure 6: Mapping of F_PT’s subfunctions on the four systems

As shown in 6.3.2, F_PT has three subfunctions (F_MM, F_CM, F_EM) for managing respectively mission, communication and energy. Where are they mapped on the four systems?

Since the three main systems are connected to the data bus they all need subfunction F_CM to handle the communication protocol. MMS is the bus controller. The two channels can manage the serial bus, but only one channel is active at a time.

The three main systems are all in charge of energy management in their own perimeter. There is no functional redundancy. EPS and HBS monitor energy consumption in their respective energy domain. They estimate the remaining electrical energy and hydraulic energy and send them to MMS. MMS aggregates, compares to what is needed to complete the mission, and makes decisions if needed.

## Communication Architecture

Figure 7: Communication architecture

Hardwired analog communications are represented in red and digital communications in green.

µXAV must meet the no single failure rule . So two paths, one digital and one analog, exist for all safety critical or mission critical information flows: sensor measurements, remaining energy capacities, actuation commands etc. The analog links are assumed resilient to very low electrical energy contrary to digital communications (so no loss of analog communications).

MMS has a single numerical link (data bus) for two-way communication with EPS and HBS. The two channels of the MMS computer are coupled to the bus and can be bus manager.

## Electrical Propulsion System

### Functional Architecture

Figure 8: Functional Architecture of the Electrical Propulsion System

At AV level the propulsion and braking energy domains are abstracted. Torques (m.N) and energy (J) for mission viability analysis are the only notions manipulated at this level. There is no need to convert torques into Volts or Amperes.

So the function in charge of this conversion energy conversion (F_PC) appears only at this level.

### Organic Architecture

It reflects the need for doubling the resources from sources to actuations:

- There are two electrical sources, the primary (whose technology is still undefined) and the secondary (Li-Ion battery). The primary is energy efficient and the secondary is power efficient. They have complementary roles and operate jointly (see Desired Behaviour section),

- The two corresponding adaptors are in charge of modulating the tension delivered by each source, depending on the objective torque at cycle k. The (tension, current) delivered at each DC motor’s ports must be such that the sum of the torques delivered by the electrical motors is as close as possible to the reference propulsion torque sent by MMS. The propulsion torque is set to zero by MMS in braking phases. The torque -> (tension, current) transduction is computed by the SoC and depends on the response curves end electrical characteristics of the motors,

- The crossbar, computer controlled switches, is unique. Its reliability (electromechanical device) is very good, far enough for the safety, reliability and availability objectives of µXAV, so there is no redundancy.

- The dual coil engine is made of two identical DC motors fixed on the same propeller shaft. They can fail independently with the same failure rate λ.

Figure 9: Architecture of the Electrical Propulsion System

## Hydraulic Braking System

### Functional Architecture

Figure 10: Functional Architecture of the Hydraulic Braking System

HBS has two specific functions:

- Braking control (F_BC). Similarly to EPS for the mechanical -> electrical transduction this function has to convert the objective braking torque into (pressure, flow) values to be submitted to the ports of the hydraulic clutch. Contrary to propulsion, the actuation device is unique and assumed failure free (strong mechanical device).

- Emergency Landing (F_EL). It has to drive the AV to zero altitude and zero speed state, preferably simultaneously. It has only braking commands available, whatever the wind conditions are. Since no energy can be supplied back to the AV, depending on wind conditions some residual speed at touchdown can’t be avoided.

### Organic Architecture

Figure 11: Architecture of the Hydraulic Braking System

Similarly to EPS most of the hydraulic devices are made redundant (2-redundancy), except the mechanical devices that are very reliable.

More detailed will be given at Modelica modelling level.

## Mission Management System

### Functional Architecture

Figure 12: Functional Architecture of the Mission Management System

The function specific to MMS is Mission Control:

- Sequencing of the phases from system initialization to the completion message sent to GS.

- Mission planning, or viability analysis: are the component failures and the remaining energy capacities compatible with mission continuation?

- Mechanical control:  computation of alternate propulsion-braking torque commands to fly the AV at requested speed / altitude / range.

MMS has no role in F_EL.

MMS contributions to F_HM (Maintenance) will be detailed in later versions.

### Organic Architecture

Figure 12: Organic Architecture of the Mission Management System

The figure is not complete .wt. communications (see Communication architecture): MMS receives the mechanical sensor values (p, pdot, q), and datas from EPS and HBS.

MS is represented apart from MMS, but as explained before, it has the same computation resources as MMS. So to be accurate MS should be represented overlaid with MMS, but then could not be distinguished from it.

## Maintenance System

### Functional Architecture

MS will have a diagnostic function and a prognostic function.

In the first version of the use case, only simple diagnostic is implemented (Boolean equations detailed at modelling level).

### Organic Architecture

MS is distributed over the EPS’ and HBS’ ECUs, and on the mission computer.

In upgraded versions of µXAV, a mass data recorder will be added for trend monitoring.

# Defined Intended Functions

## Introduction

This part is assumed to be the capture of the Desired Behaviour, at AV level and then at System Level. Information is expected to be enough precise and complete to make implementation possible at item level. Mapping of the system requirements on the items is done in this section.

Handling OP Correctness verification on the use case’s DIF and implementation would be facilitated by separating the declarative parts from the constructive ones in the DIFs. The goal is to delineate the three different kinds of Correctness verification, from a conceptual and tooling perspective:

- set inclusions (property –> property),

- satisfiability relations (behaviour -> property),

- behaviour inclusions (behaviour -> behaviour).

So a DIF is described in three paragraphs:

- Design Rational: the why of the functional architecture, of the properties and imposed behaviours (solution-oriented specifications),

- Declarative part: the behavioural properties that can be compared to upper level properties (narrowing of the space of conformant behaviours) or evaluated on concrete executions (satisfiability relation). This is the contract part of the DIF (the what). The contract is split in two parts:

  - Declared Assumptions on the inputs of the function or on aspects of its environment that may influence its behaviour,

  - Expected Guarantees the input-output relations to be ensured to meet the desired behaviour,

- Constructive part: any specification trait similar to a behaviour, described operationally (the how): a sequence of actions, computations, an algorithm, a mathematical function etc. It constructs values that must satisfy the contract.

## Air Vehicle Level

### Payload Transport

#### Interface

The I/Os are that perceived by the mission operator.

The notation <type>.<unit> is used for physical values to indicate:

- the type for typing consistency,

- the IS unit, for physical consistency

| Inputs | From | Outputs | To |
| --- | --- | --- | --- |
| NavigationParameters( [Distance:Integer.m; Altitude:Integer.m; Speed:Integer.(m/s)]) | GS | MissionCancelled(Boolean) | CP |
| NavigationMode({‘A’,’RP’}) | GS | MissionComplete(Boolean) | CP |
| NavigationOption({‘SPEED’,’ENERGY’}) | GS | MissionAborted(Boolean) | CP |
| PowerSwitch({‘ON’,’OFF’}) | CP | PrimarySourceDisplay(Integer.%) | CP |
| NavigationMode({‘A’,’RP’}) | CP | SecondarySourceDisplay(Integer.%) | CP |
| PayloadBay(‘OPENED’,’CLOSED’) | CP | LEDi(Boolean) i in {1:20} | CP |
| StartButtonPress(Booelan) | CP | PropulsionTorque(float.(N.m)) | AV’s body |
| PayloadMassRotactor1({‘1’,’2’,...,’9’}) | CP | BrakingTorque(float.(N.m)) | AV’s body |
| PayloadMassRotactor2({‘1’,’2’,...,’9’}) | CP | EmergencyLanding(Boolean) | AV.F_EL |
| USBkey( [Mass:Integer.kg; Distance:Integer.m; Altitude:Integer.m; Speed:Integer.(m/s)]); | CP |  |  |
| MechanicalSensors(p,pdot,q) | AV’s body |  |  |
| PrimaryEnergyLevel(float.J) | EPS. F_EM |  |  |
| SecondaryEnergyLevel(float.J) | EPS. F_EM |  |  |
| EquipmentsOK(Boolean) | AV.F_HM |  |  |

#### Design Rational

The reactive part (modes, events) is made of the simplest phase logic: take-off, climb, cruise, descent, landing, stop. See Figure 14. Many variants of the nominal sequence are possible. For instance:

- Climb, then descent without cruise (e.g. soft landing because of icing in climb),

- repeated (climb;cruise) and/or repeated (descent;cruise),

There is logical independence between GS controls and MMS controls:

- navigation options can be modified at any time by the GS operator,

- the navigation options are left unchanged by all the phase transitions.

The AV’s behaviour is insensitive to option changes during take-off and landing (too short phases).

There is a stratified approach to AV’s control laws:

- Upper level: F_PT at AV level (AV.F_PT.F_MM) is not aware of the energies powering propulsion and speed braking. F_PT.F_MM computes in the mechanical domain,

- Lower levels: EPS and HBS have to convert the mechanical requests (torque commands) into electrical or hydraulic actions.

AV.F_PT may use:

- Propulsion-only control,

- Braking-only control,

- Joint (propulsion + braking) control. For instance “bang-bang” policy to maximize speed at safety envelope boundary in poor weather conditions (requires the SPEED option).

AV.F_PT activates the option-dependent control laws to meet the following ranked objectives:

- cover the distance at prescribed altitude and speed,

- If not possible, relax the speed constraint and enter energy-saving mode,

- If still not possible (altitude + distance + weather conditions are incompatible with onboard energy) activate soft landing.

#### Declarative Part

##### Assumptions

- Foreseable operating conditions for wind gusts are the regulatory ones,

- Foreseable operating conditions for icing are the regulatory ones,

- Standard Weather Conditions (SWC) are defined by ATC standards (TBDL at model level)

- Good Weather Conditions (GWC) are defined by no icing and tailwind (no vertical component) of strength less than 15kt, distributed as uniform law [0,15],

- Adverse Weather Conditions (AWC) are defined by no icing and headwind with some vertical component. AWC wind is characterized by:

  - Horizontal headwind:

    - speed fluctuates in [10kt, 25kt] distributed as GL(18,5),

    - direction with respect to heading fluctuates in [-π/4, +π/4] uniformly,

  - Vertical headwind: strength fluctuates in [-10kt, +20kt] as GL(10,5),

- Operators may enter any value by means of the GS channel or the USB channel (no assumption on the values for robustness reasons). But the values are assumed to be well-typed and appropriately bounded (needed to discard some security attack aspects).

Note: On Gaussian laws (infinite support) a finite support is added for worst case analyses in control law design or dimensioning of physical devices. The worst cases are defined by the bounds of the finite support. Their occurrence can be quantified by the normal law.

##### Guarantees

- No take-off if onboard energy is incompatible with the mission, under SWC assumption,

- Climb performance: ascending speed kept in [0.2, 3] m/s, precision of flight level capture within  +- 5% of the reference value,

- AV kept in the safety flight envelope (a phase-dependent constraint on p, pdot, q, qdot),

- RP mode is identical to A mode when no command is sent to the AV. All control laws and FDIR behaviours are left invariant by ‘A’↔‘RP’ switches,

- In ideal weather conditions (no wind, no icing), in case of at most 1 failure per component type, all initiated missions run to completion,

- In GWC, at MTOW, when the range is lower than 100 n.m, the mission success ratio is at least 90% (95% confidence level),

- In AWC, at MTOW, when the range is lower than 50 n.m, the success ratio is at least 60% (95% confidence level),

- As soon as rejection of external disturbances (safety envelope) or mission completion is no longer possible, soft landing should be triggered,

- As soon as rejection of external disturbances (safety envelope) or mission completion is no longer possible and propulsion is no longer available, hard landing (F_EL) should be triggered,

#### Constructive Part

It decomposes into six mission stages and two modes (‘RP’, ‘A’). The stages are common to both modes. A hierarchical state machine specifies the sequencing logic (Fig 14).

The stages are:

- Payload loading: initiated by opening the bay after putting the CP’s switch in OPENED position. Mass of the payload must be given to the system (GS message or CP’s rotactors, or USB key).

- Mission set-up: must be given prior to take-off:

  - Transport distance (km),

  - Cruise speed (m/s),

  - Flight level (m)

The Desired Behaviour refers to aeronautical units (knots, nautic miles, feet) but DIFs, design and implementation use IS units (m, kg, s).

Figure 14: Higher Level Activation Logic

- Take-off: after payload loading and mission set-up. Initialization of the systems must have succeeded (READY lighted on). Press of CP START button or GO message from GS needed.

- Cruise: MMS strives to keep altitude and speed close to the preset values in spite of the atmospheric disturbances. AV.F_MM uses thrust or brake pulses, or modulated continuous actions.

  - Altitude is defined by a function of p, pdot and some AV’s parameters.

  - Speed is derived from p, pdot, qdot and some AV’s parameters,

  - The covered distance is computed from p, pdot and some AV parameters,

The three functions will be supplied at model level.

Once the target range is covered, normal landing is initiated.

- Landing: performs simultaneous speed and altitude decrease (p, pdot) while trying to maintain q and qdot as close to 0 as possible, until immobility is reached,

- Payload unloading: the OPEN/CLOSED switch is activated again. End of the mission cycle.

RP and A modes are similar. RP is identical to A when the GS operator sends no command to the AV. RP gives the opportunity to modify the mission parameters (altitude, speed, distance) at any moment. This is the only way of remotely ‘guiding’ the AV. Remote piloting is impossible because of excessive communication latency (+ lack of sensors).

Robustness of AV.F_PT: it has to cope with external and internal disturbances.

External disturbances:

- Wind conditions: fluctuating 3D wind vector. Can accelerate or slow down the AV, or have it quit the prescribed altitude. It can lead to electrical energy saving or over consumption.

- Icing conditions: temporary or persistent. Modelled as mass perturbation. MMS control laws have to detect and estimate the possible increase/decrease of AV’s mass.

Internal disturbances:

- Energy run-out: should never occur because of faulty initial conditions (mission preparation), i.e inconsistency between the onboard energy level and the mission to complete (under SWC assumption). Energy run-out may happen because of en-route unexpected weather conditions,

- Breakdowns: the multi-system architecture is designed to tolerate all single physical item failures. Some combinations of failures can’t be tolerated. F_EL has to be triggered before AV becomes uncontrollable.

### Subfunctions of Payload Transport

F_PT is unique in being decomposed at a lower level. Possibly F_HM will also be decomposed in later versions.

It provides some functional and architecting complexity to exercise FDAL/IDAL assignment on the safety process side (ARP 4761A Appendix P).

#### Mission Management (AV.F_PT.F_MM)

##### Interface

There are redundant input channels to load the mission parameters: GS messages and CP devices (USB key + rotactors and switches). The payload mass may be supplied thrice (with inconsistencies), twice or once.

F_PT.F_CM (Communication Management) is in charge of handling these redundancies. It builds a unique up-to-date value for all sensors or parameters needed by mission management (reminder: F_MM ensures the FCS-like and GNC-like functions). The event-driven inputs are converted into continuous time data for F_PT.F_MM. It is necessary for making F_PT.F_MM a pure control function.

Conversely, F_PT.F_MM posts its outputs to F_PT.F_CM for dispatch to the external devices (GS, CP).

| Inputs | From | Outputs | To |
| --- | --- | --- | --- |
| NavigationParameters( [Distance:Integer.m; Altitude:Integer.m; Speed:Integer.(m/s)]) | F_PT.F_CM | MissionCancelled(Boolean) | F_PT.F_CM |
| NavigationOption(‘SPEED’,’ENERGY’) | F_PT.F_CM | MissionComplete(Boolean) | F_PT.F_CM |
| NavigationMode({‘A’,’RP’}) | F_PT.F_CM | MissionAborted(Boolean) | F_PT.F_CM |
| PowerSwitch({‘ON’,’OFF’}) | F_PT.F_CM | PropulsionTorque(float.(N.m)) | AV’s body |
| PayloadBay(‘OPENED’,’CLOSED’) | F_PT.F_CM | BrakingTorque(float.(N.m)) | AV’s body |
| StartButtonPress(Booelan) | F_PT.F_CM | EmergencyLanding(Boolean) | F_EL |
| PayloadMass(Integer.kg) | F_PT.F_CM |  |  |
| MechanicalSensors(p,pdot,q) | F_PT.F_CM |  |  |
| EnergyLevel(J) | F_PT.F_EM |  |  |
| RessourcesOK(Boolean) | F_HM |  |  |

##### Design Rational

F_PT.F_MM is in charge of phase sequencing (AV’s dynamics .wt. to navigation constraints) and of the control laws. They were described at F_PT level and are mapped on this subfunction.

F_PT.F_MM receives the energy conditions and failure modes from F_PT.F_EM and F_PT.F_HM.

No use of “defensive specification” (analogous to “defensive programming” in software). See F_MM’s Assumptions.

##### Declarative part

The safety and performance properties presented at F_PT level that can be formulated on F_PT.F_MM’s I/O and state variables are not repeated here. So to speak, this part of F_PT.F_MM’s contract is the projection of F_PT’s contract on F_MM’s interface.

F_PT.F_MM’s contract contains also additional properties, specific to the exchanges with F_PT.F_EM and F_PT.F_CM. These exchanges are unobservable on F_PT’s interface.

###### Assumptions

- RessourcesOK is correct, i.e never misleading. When true, propulsion, braking, computation and communication and should actually be available,

- EnergyLevel(J) is accurate or conservative. If conservative, not in excess by 10% of the estimated capacity.

- Icing can occur only in climb and descent phases (no icing at µXAV’s cruise altitudes),

Because of:

- systematic contract-based specification (the DIFs),

- systematic contract-based implementation (system modelling, software and hardware development),

- an assumption on sufficient effectiveness of development assurance,

- assumptions A and B,

F_PT.F_MM has no mitigation mechanism to tolerate faults that would lead to a wrong perception of µXAV’ state. F_PT.F_MM does not try, using estimation algorithms, to reconstruct energy or availability information on its own in order to cross check the data it receives.

In the use case, there is no functional redundancy motivated by possible weakness of development assurance.

If functional redundancy had to be introduced (none in the first version), it would be on request of the safety group, for architectural reasons.

###### Guarantees

Those derived from projection of F_PT’s guarantees on F_PT.F_MM’s interface (should be tool-supported). They are the correctness properties of the control laws (FCL_xxxx algorithms of the next section).

##### Constructive part

It consists in the statechart (fig 14) and in the control algorithms activated in each of its state. There are 1 + 3*2 + 1 = 8 flight control laws:

- FCL_TakeOff: a simple PI controller to ramp-up p and pdot to the thresholds while maintaining q=qdot=0. No disturbance rejection. The phase lasts a few seconds.

- FCL_ClimbEnergy: constant vertical speed, constant horizontal acceleration until cruise speed. Disturbance rejection only in the vertical plane. Optimal criterion on propulsion energy. No braking energy unless AV escapes the safety envelope. Mass estimation, emergency landing if the upper threshold is crossed. The phase lasts minutes (altitude dependent),

- FCL_ClimbSpeed: PI on maximal horizontal speed and vertical speed. Braking only in case of safety escapes. Mass estimation and F_EL trigger logic are active,

- FCL_CruiseEnergy: optimal control (energy criterion) to stay at cruise altitude and horizontal speed. No icing detection (assumption C),

- FCL_CruiseSpeed: PI on maximal horizontal speed compatible with the disturbances (rear wind). No icing detection (assumption C),

- FCL_DescentEnergy: similar to climb, with different control parameters (see model),

- FCL_DescentSpeed: identical to FCL_ClimbSpeed, with different control parameters,

- FCL_Landing: ensures simultaneous zeroing of the 4 state vector components.

#### Energy Management (AV.F_PT.F_EM)

##### Interface

At AV level, from the operator or maintainer standpoints, energy management is almost fully abstracted. The maintainer, because of the CP’s 20 LEDs, is aware of the architecture, in particular the electrical/hydraulic aspects.

The AV level DIF of F_EM addresses the operator’s point of view, not the maintainer’s. So energy management at AV level is just a matter of global level to be compared to an estimate of what is needed to complete the remaining part of the mission. It is abstracted from the sources except from the fact that there are two sources

At system level, once F_EM is mapped on EPS, MMS and HBS, it is refined with the specification traits relevant at this level.

| Inputs | From | Outputs | To |
| --- | --- | --- | --- |
|  |  | EnergyLevel(float.J) | F_PT.F_MM |

##### Design Rational

AV.F_PT.F_EM uses two internal informations:

- PrimaryEnergyLevel(float.J)

- SecondaryEnergyLevel(float.J)

Energy management is so abstracted that one can only add the two capacities.

At lower levels the two sources appear with different response profiles: the primary is energy oriented (slow, not very powerful but lasting) while the secondary is power oriented (responsive, powerful but quickly exhausted). They act in a joint way. The aggregation law of their respective capacities is more complex than F_PT.F_MM’s mere addition.

##### Declarative part

###### Assumptions

- PrimaryEnergyLevel(float.J) is accurate or conservative (at most by 10% at 95% CL)

- SecondaryEnergyLevel(float.J) is accurate or conservative (at most by 20% at 95% CL)

###### Guarantees

EnergyLevel(float.J) is accurate or conservative (at most by 20% at 95% CL)

##### Constructive part

As no information is available, summation of the two capacities.

#### Communication Management (F_PT.F_CM)

##### Interface

At AV functional level (AV like a black box), there is no awareness of system-to-system communications. Communications between the three functions are “for free”.

Following the operator’s standpoint, the communications considered are that with the external devices: GS and CP. From the point of view of the three functions, CP is regarded as a peripheral device.

| Inputs | From | Outputs | To |
| --- | --- | --- | --- |
| GS_MissionSetUp( [Distance:float; Speed:float; Altitude:float]) | GS | AV_CP_Displays( [READY:Boolean; CANCELLED:Boolean; COMPLETE:Boolean; ABORTED:Boolean; PSRC_CAPACITY:Integer; SSRC_CAPACITY:Integer ]) | CP |
| GS_NavigationMode({‘RP’,’A’}) | GS | AV_CP_LEDs(bitarray[20]) | CP |
| GS_NavigationOption({‘SPEED’,’ENERGY’}) | GS | AV_MissionCompleted([PRIMARY_SOURCE:Integer; SECONDARY_SOURCE:Integer; ]) | GS |
| GS_GO | GS |  |  |
| GS_EmergencyLanding | GS |  |  |
| AV_CP_Switches( [Power: Boolean; Mode: {‘RP’,’A’}; Bay:{‘OPEN’,’CLOSED’}; START:Boolean; Rotactor1:Integer; Rotactor2:Integer;} ]) | CP |  |  |
| Analog signals from the three mechanical sensors (p, pdot, q) | AV |  |  |

##### Design Rational

The use case abstracts the protocol of the data link with the ground station.

The link may fail, but there is no message loss when it works. Communications are event-driven (i.e on demand, not periodic) with message queuing at both ends (asynchronous communications).

##### Declarative part

###### Assumptions

- The data-link is not a lossy channel,

- Communication latencies are bounded (2s),

- Transfer preserves integrity of contents (error correcting codes – no security aspects)

- The hardwired links with the mechanical sensors can’t fail

###### Guarantees

That of the assumptions (integrity, no loss, bounded delay),

##### Constructive part

F_PT.F_CM ensures µXAV’s send and receive operations, in conformance to grouping of data into messages (see Interface).

It also performs the digital encoding of the sensors’ analog values.

These data are made available for, or read from, F_PT, F_EL, F_HM.

F_EL uses only the mechanical sensors. It sends no information.

In case of transitioning from F_PT to F_EL, message AV_MissionCompleted is sent just before entering F_EL. F_EL is not considered as another phase of the mission, but as termination of the mission in a few seconds (~ [20s, 40s]).

### Emergency Landing

F_EL is also named “hard landing”.

It has to be robust to complete loss of electrical power, through exclusive use of braking actions.

AV’s final position may not satisfy p=0, deceleration may be excessive, with possible impact on the payload and AV’s airworthiness potential.

#### Design Rational

F_EL degrades AV’s mechanical energy (kinetic and potential) as quickly as possible to zero altitude and zero motion.

F_EL may be triggered by the three systems MMS, EPS and HBS (depending on the breakdowns or violent gusts), but by only one of them at a given time. To this end the three systems receive the analog and digital values of AV’s state vector (p, pdot, q, qdot) to check if it stays in its safety-domain. This hot redundancy is necessitated by seamless FDIR transients of AV’s dynamics.

As emergency landing is irreversible, the three systems cross-check their respective assessments, a 2-over-3 consolidation logic is applied when all systems are up. When only two are up, the master decides (priorities: MMS > EPS > HBS).

For instance, if MMS is down while EPS and HBS are operational and the two decisions are different, EPS overrides HBS.

F_EL has no mode or options.

#### Declarative Part

##### Assumptions

- No assumption. F_EL is the ultimate recovery process to avoid crash, its robustness has to be maximal,

##### Guarantees

- No false negative of the activation logic, i.e loss of the AV whereas anticipation was possible. The trade-off between the success ratio and the energy margins is at stake here. The lower the margins, the higher the mission success ratio, and the risk of catastrophic events,

- No false positive of the activation logic, or more likely a ratio that complies with the prescribed success ratio,

- Under assumptions of enough left energy and SWC, no damage to the AV (specified as dynamic constraints on the state vector).

#### Constructive Part

An optimal control algorithm: fastest descent compatible with the flight envelope, at minimal hydraulic energy consumption.

### Health Management

#### Interface

| Inputs | From | Outputs | To |
| --- | --- | --- | --- |
| PropulsionRessourcesOK(Boolean) | EPS | RessourcesOK(Boolean) | F_PT.F_MM |
| BrakingRessourcesOK(Boolean) | HBS |  |  |
| MissionManagementRessourcesOK(Boolean) | MMS |  |  |

#### Design Rational

At AV level the internal structure of the systems (pumps, ECUs, battery etc.) is abstracted. So the behaviour of the function is restricted to the synthesis part that computes RessourcesOK(Boolean) for F_PT.F_MM.

There are two kinds of data scanned and processed by F_HM:

- The failure statuses sent by the computerized items (MMS.ECU (x2), EPS.ECU (x2), HBS.ECU (x2)),

- The analog signals that carry physical measurements (mechanical, electrical, hydraulic sensors) or discrete states (e.g CP).

The outputs of the function are high level functionalized failure statuses. Functionalized means that F_HM gives to MMS the functions that are lost:

- Payload transport at AV level,

- Propulsion, braking, mission control at system level.

Health management does not address energy level issues.

#### Declarative Part

##### Assumptions

- Sensors are ideal. In later versions the sensors will include fault models (bias, noise, lag) to deal with robustness analysis at AV and system level,

- Actuators are ideal. Similarly in later versions the actuators will be biased and lagging,

- Some item level components have no failure modes (mechanical parts, wiring, pipes, see Architecting),

- The synthesis information from EPS, HBS, MMS are accurate or conservative. Conservative means that if wrong, it is in a way that does not compromise safety. xxxxOK(false) while everything is yet OK leads to unnecessary grounding of µXAV. It should happen only on rare situations unavoidable by design at reasonable cost. So wrongness, provided it is mastered by design, may be correct as far as OP Correctness is concerned. This kind of situation may happen for instance during transients (mode switches, failures, gusts) where sensors lag .wt. actual physics, leading to inconsistent diagnostics and consequent inappropriate decisions.

##### Guarantees

- No false negatives (safety critical). The drone cannot be dispatched in case of known unrepaired components. This is because the architecture is only 2-redundant. In case of latent failures at take-off, a single in flight additional failure may lead to a catastrophic event, which is banned by the regulation.

- No false positives (mission critical). Nice to have, though classified minor by FHA (bears only on availability).

- RessourcesOK is accurate or conservative.

#### Constructive Part

F_PT ressources are OK as long as propulsion, braking and mission management resources are OK.

## System Level

### Introduction

Two systems are pure software systems hosted in the same computer (MMS and MS), and two systems are CPS, i.e controlled physics (EPS and HBS). In all cases there is only one “smart unit” (or ECU) that contains “intelligence” and implements the control behaviour specified as ‘DIF’.

Proceeding in the functional refinement of µXAV, there is no ambiguity in structuring the DIF as:

- EPS.F_PT.F_PC : Propulsion Control

- EPS.F_PT.F_EM : Energy Management

- EPS.F_PT.F_CM : Communication Management

- EPS.F_HM : Health Monitoring

- HBS.F_PT.F_BC: speed Braking Control

- HBS.F_PT.F_EM: Energy Management

- HBS.F_PT.F_CM: Communication Management

- HBS.F_EL: Emergency Landing (also called hard landing)

- HBS.F_HM: Health Monitoring

- MMS.F_PT.F_MM: Mission Management (includes GNC aspects)

- MMS.F_PT.F_EM: Energy Management

- MMS.F_PT.F_CM: Communication Management

- MMS.F_HM: Health Monitoring

- MS.F_HM: Health Monitoring

The reason for unambiguous mapping of these functions on the systems’ components is that they are mapped on the unique ‘smart unit’ contained by the system they are mapped on.

Two functions are not visible at AV level, and appear at system level (internal view of the systems, located in them):

- F_PC, the Propulsion Control Function specific to EPS’ controller,

- F_BC, the speed Braking Control Function specific to HBS’ controller.

MS (Maintenance System) is composed of EPS.F_HM + HBS.F_HM + MMS.F_HM.

The DIF descriptions of these 14 functions are structured like the AV level DIFs.

### EPS’ Functions

#### Propulsion Control (EPS.F_PT.F_PC)

##### Interface

EPS’ ECU has both analog and digital links handled by F_CM. Analog links are never power links, they are informational links intended to be resilient to MMS losses (e.g soft landing mode).

F_PC communicates through the I/O variables that are listed here (F_CM in charge):

| Inputs | From | Outputs | To |
| --- | --- | --- | --- |
| PropulsionTorqueCommand(float.J) | EPS.F_PT. F_CM | Motor1VoltageCommand(float.V) | EPS.F_PT. F_CM |
|  |  | Motor2VoltageCommand(float.V) | EPS.F_PT. F_CM |
| FBmotor1(Boolean) | EPS.F_PT. F_HM | SwitchConfiguration({‘1-12’,’1-1’,’1-2’,’2-12’,’2-1’,’2-2’}) | EPS.F_PT. F_CM |
| FBmotor2(Boolean) | EPS.F_PT. F_HM | PropulsionOK(Boolean) | EPS.F_PT. F_CM for MMS.F_PT.F_MM |
| FBswitch(Boolean) | EPS.F_PT. F_HM | Pump1PowerSwitch(Boolean) | EPS.F_PT. F_CM |
| PEPSvoltageSensor(float.V) | EPS.F_PT. F_CM | Pump2PowerSwitch(Boolean) | EPS.F_PT. F_CM |
| PEPScurrentSensor(float.A) | EPS.F_PT. F_CM |  |  |
| SEPSvoltageSensor(float.V) | EPS.F_PT. F_CM |  |  |
| Motor1VoltageSensor(float.V) | EPS.F_PT. F_CM |  |  |
| Motor2VoltageSensor(float.V) | EPS.F_PT. F_CM |  |  |
| pSensor(float.rad) | EPS.F_PT. F_CM |  |  |
| pdotSensor(float.(rad/s)) | EPS.F_PT. F_CM |  |  |
| qSensor(float.rad) | EPS.F_PT. F_CM |  |  |
| PEPSestimatedCapacity(Integer.%) | F_PC |  |  |
| SEPSestimatedCapacity(Integer.%) | F_PC |  |  |

F_PC is exclusively in charge of motors’ control (see Implementation section for more details on motors). It relies on F_EM and F_HM for resource awareness. F_PC is in charge of FDIR logics that enforces the fail operational/fail safe policies.

##### Design Rational

Modes

Two propulsion modes:

- MMS Master: F_PC operates only as some sort of software-based energy transducer: it converts N.m torques into (V, A) motor commands. It does not use the mechanical sensors though permanently made available by EPS.F_PT.F_CM. AV’s dynamics is exclusively controlled at MMS level in this mode,

- EPS Master: when MMS’ both channels are lost, and the data bus consequently. In this failure condition the mission is aborted, µXAV‘s soft landing is engaged if electrical energy is OK. If not OK, emergency landing is engaged (see HBS.F_EL).

Soft landing is slow decay of the drone’s energy, by propulsion only control (if no wind gusts require urgent braking actions). Descent may take about ten minutes contrary to emergency landing which lasts a few tens of seconds. When waypoints are introduced (upgraded version of the use case), soft landing will control AV’s navigation to stop the aborted mission at the nearest way point (not in the fields at random like F_EL does).

In soft landing, since there is no longer MMS’ thrust-brake coordinated actuation, HBS acts on its own when keep the AV in its safety envelope requires brake pulses (e.g rear wind induced over speed).

Source Balancing

SEPS is responsive, powerful, but not long lasting. PEPS is durable but not powerful enough to cope with worst case torque commands. So F_PC has to pilot SPES as a reserve when PEPS runs out, but also as a supplement in highly demanding transients. SEPS is not designed to supplement PEPS in steady states (quiet cruise conditions).

Actuation balancing

The DC motors are redundant. Both of them are capable of coping with worst case torque demands. F_PC is in charge of using the twin motors at best (performance, endurance, FDIR, etc.)

Control

F_PC receives the propulsion torque commands by means of the data bus (10Hz periodic messages). There is no jitter, unless the bus master commutes from MMS.ECU1 to MMS.ECU2 (failure of ECU1). AV’s stability in such transients ([300, 800ms]) is a design issue.

EPS’ control law is unique. It is used in any mission phase and in any failure condition. It consists in commanding the right voltage(s) at motor(s) inlet so that the resulting torque is equal (+- precision bound) to the commanded torque.

FDIR

The computer controlled power switch (also named crossbar, the two words are synonymous) is in charge of reconfiguring the power links from sources to motors. This power switching logic ensures the highest availability of propulsion.

FDIR also monitors the electrical network’s stability (oscillations, collapses, etc.). In case of instabilities correlated with brake commands it can decide to isolate (and so to power-off) the active hydraulic pump. HBS has its own sensors to detect such an event, without notice by EPS.

Hydraulic pumps’ power supply

EPS supplies the two pumps with non modulated voltage. HBS is in charge of modulating, inside the pumps, the electrical power inlet so that the pumps deliver modulated pressure. Electrical power delivered to the two pumps is permanent and identical in normal situations. It is taken from the primary source until PEPS’ tension falls under a threshold PVmin. Then it commutes to SPES.

As cavitation may have nasty electrical influences (current draining), EPS can individually isolate the pumps from the electrical network (without notice to HBS).

##### Declarative part

###### Assumptions

- Propulsion torque commands are correct,

- Electrical sensors are accurate (to be discarded in later versions),

- Mechanical sensors are accurate (to be discarded in later versions),

- Motors are ideally conformant to their specification (to be discarded in later versions),

###### Guarantees

- Electrical commands meet the motors’ specifications (performance and safety domains TBD at model level),

- Torque is conserved in the transduction from mechanical to electrical,

- In case of single component failure, propulsion is sustained.

- The two electrical pumps are permanently and identically supplied, unless cavitation induced disturbances are detected,

- Any pump diagnosed as draining current for more than 3s, is isolated from the network.

##### Constructive part

Defensive design: µXAV’s designers and implementers trust OPs-based development assurance: no defensive design. F_PC could use the mechanical sensors to build a consistency check on the torque command .wt. AV’s position in the safety envelope. But F_MM’s correctness is assumed to be guaranteed.

And F_CM guarantees integrity of the command transfer.

If some functional (independent) redundancy had to be introduced, it would be on explicit request of the safety group, not on EPS’ designers own initiative by confused fear of loopholes.

Conversion law

Will be given when a physically consistent model is operative. Will use the motors’ characteristic curve provided by the supplier.

Switching law

When both motors are operative both are symmetrically active (half power each), otherwise the failed motor is isolated (by a specific isolation switch).

PEPS is the only source connected to the network, except in the following situations:

- Power on and systems boot: supplied by SEPS only, until PEPS completes its ramp-up and reaches nominal tension (duration: [1mn, 3mn], temperature dependent),

- Over demanding propulsion transients requiring SEPS’ additional supply,

- PEPS’ capacity falls under the minimum acceptable level (PEPScapacityLowerBound).

Pump monitoring

There are no current sensors to detect excessive current demand. Excessive current is associated with slight tension drops at pump input port. When these slight drops are temporally correlated with braking commands, they are used as electrical signatures of cavitation. They trigger pump isolation.

#### Energy Management (EPS.F_PT.F_EM)

##### Interface

| Inputs | From | Outputs | To |
| --- | --- | --- | --- |
| PEPSvoltageSensor(float.V) | EPS.F_PT. F_CM | PEPSestimatedCapacity(Integer.%) | F_PC, F_HM |
| PEPScurrentSensor(float.A) | EPS.F_PT. F_CM | SEPSestimatedCapacity(Integer.%) | F_PC, F_HM |
| SEPSvoltageSensor(float.V) | EPS.F_PT. F_CM |  |  |
| BrakingTorqueCommand(float.(N.m)) | EPS.F_PT. F_CM |  |  |

##### Design Rational

One assumes that EPS’ ECU knows the exact initial capacity of the two sources. Energy consumption has to be estimated to be periodically subtracted to the initial capacities. For both sources three independent estimation methods are used. The estimates are then conditionally fused to get a unique robust estimate per source:

- Method1: use of the supplier’s estimation function: it takes as input the outlet tension of the source in zero-load situations, and outputs an estimate of the remaining capacity. This is an empirical curve given as a table look-up. It necessitates detecting “zero-load situations” because source outlet tension is a reliable capacity indicator only in open circuit situations. “Zero-load situations” never occur in an absolute sense during flight. Minimal or low demand situations are used as so-called “zero-load situations”. In particular it is the case in “reversal situations”: propulsion drops to minimum and braking starts, or braking stops and propulsion resumes. These two kinds of low load transients, whose duration ranges from 100 ms to 1s, are sampling windows for capacity estimation. EPS’ ECU uses the torque commands (propulsion + braking) to detect these reversal phases. Such phases may never occur, in particular in GWC.

- Method2: converts the requested mechanical energy (effort x displacement) into requested electrical energy. This requires using the motors’ datasheets (in particular its yield). Integration over time of the theoretical requested electrical energy, subtracted to initial capacity yields an estimate of the remnant capacity,

- Method3: the energy actually supplied can be estimated by means of the electrical sensors at source and motor level, instead of a theoretical conversion from mechanical to electrical. But the measured consumption over time includes that of the active hydraulic pump, which cannot be isolated and discounted. Moreover in case of joint supplies, PEPS’ and SEPS’ deliveries may be non separable for reconstructing their proper contributions.

##### Declarative part

###### Assume

- EPS.F_PT.F_EM has an accurate estimation of PEPS’ and SEPS’ initial capacities at power-on (no uncertainty interval). The two constants are PEPSinitalCapacity(float.J) and SEPSinitalCapacity(float.J),

- The tension-capacity curves of PEPS and SEPS loaded in the ECUs’ memory (PDIs) are accurate .wt. the actual source items of the architecture. In particular the two uncertainty curves are accurate as well (diameter dependent of the operating point),

- Tension and current sensors deliver accurate measurements (to be modified later – fault model TBD),

###### Guarantee

- F_EM computes accurate estimation intervals, and sends the centre of each interval as the best estimate (then converted into % of the initial capacity).

- F_EM guarantees the uncertainty intervals so that MMS can use them for risk margin management. This is critical when energy exhaustion at mission completion is forecast only for a small part of the uncertainty interval.

##### Constructive part

There are two algorithms running in parallel for method2 and method3, from power on to power off. These algorithms are sketched in the rational section and will be supplied at model level. The algorithm of method1 is activated on event, when a so called “zero-load” window has been detected. A circular buffer stores the measurements over a 5s sliding window.

A fourth algorithm is in charge of fusion of the two or three estimates. The fusion logic is the following:

- When reversal transients occur with duration greater than 500ms, the so-called “zero-load” snapshot estimates (method1) are the most reliable ones. They are used as best estimates with uncertainty given by the suppliers’ uncertainty curves. Any such snapshot estimate resets the current estimates of both sources (validated value). No fusion with method2’s and method3’s estimates,

- When reversal transients are shorter than 500ms but longer than 100ms, the three estimates are averaged. The estimates are also reset with these new values.

- If, since the last reset, hydraulic activity was low or null, the two estimates are updated by methods 2 and 3, and 3 is still reliable because of low electrical pump activity. F_EM needs F_PC’s source balancing logic to know if PEPS acts alone or reloads SEPS, or supplies jointly with SEPS. Measured electrical consumption has to be allocated to the two sources. This allocation process needs an inversion algorithm (TBD at model level). For each source one averages its estimated consumptions by method2 and method3 with respective weights (1,2),

- If hydraulic activity is significant since the last reset (decision is made using the braking commands received by EPS on the bus), the algorithm is the same as previously except the weights which are (2,1) instead of (1,2). This is because HBS’ contribution in energy consumption becomes significant in the measurements and cannot be reconstructed for discounting.

#### Communication Management (EPS.F_PT.F_CM)

##### Interface

The interface at ECU equipment level is made of an analog part and a digital part, both managed by the F_CM subfunction. The design is similar to that of MMS.ECU and HBS.ECU.

MMS.F_PT.F_MM, EPS.F_PT.F_PC and HBS.F_PT.F_BC have in common a direct analog link to the mechanical sensors. It is necessitated by autonomous control of the drone’s in case of MMS’ unavailability.

EPS.F_PT.F_CM ensures digital encoding of the analog data and the send/receive operations on the data bus.

###### Analog

| Inputs | From | Outputs | To |
| --- | --- | --- | --- |
| PEPSvoltageSensor([0,270V]) | EPS.F_PT. F_CM | Motor1VoltageCommand([0,28V]) | Voltage Modulator |
| SEPSvoltageSensor([0,270V]) | EPS.F_PT. F_CM | Motor2VoltageCommand([0,28V]) | Voltage Modulator |
| PEPSCurrentSensor([0,50A]) | EPS.F_PT. F_CM | SwitchConfiguration({‘1-12’,’1-1’,’1-2’,’2-12’,’2-1’,’2-2’}) | Switch |
| Motor1VoltageSensor([0,270V]) | EPS.F_PT. F_CM | HydraulicPump1PowerSupply([0,28V]) | EPS. SwitchPump1 |
| Motor2VoltageSensor([0,270V]) | EPS.F_PT. F_CM | HydraulicPump2PowerSupply([0,28V]) | EPS. SwitchPump2 |
| pSensor [0,28V] continuous signal encoding [-π,+π] | AV |  |  |
| pdotSensor [0,28V] continuous signal encoding [-π20/s,+20π/s] | AV |  |  |
| qSensor [0,28V] continuous signal encoding [-π,+π] | AV |  |  |

Voltage commands are not power signals (0-28V). They provide the modulated value to be supplied by the power modulator, not the modulated power itself.

###### Digital

| Inputs | From | Outputs | To |
| --- | --- | --- | --- |
| PropulsionTorqueCommand(float.J) | MMS.F_PT.F_MM | PropulsionTorqueCommand(float.J) | EPS.F_PT. F_PC |
| Motor1VoltageCommand(float.V) | EPS.F_PT. F_PC | pSensor(float.rad) | EPS.F_PT. F_PC, F_HM |
| Motor2VoltageCommand(float.V) | EPS.F_PT. F_PC | pdotSensor(float.(rad/s)) | EPS.F_PT. F_PC, F_HM |
| SwitchConfiguration({‘1-12’,’1-1’,’1-2’,’2-12’,’2-1’,’2-2’}) | EPS.F_PT. F_PC | qSensor(float.rad) | EPS.F_PT. F_PC, F_HM |
| PropulsionOK(Boolean) | EPS.F_PT. F_PC | PropulsionOK(Boolean) | MMS.F_PT.F_CM |
| BrakingTorqueCommand(float.(N/m)) | MMS.F_PT.F_CM | PEPSvoltageSensor(float.V) | EPS.F_PT. F_EM, F_HM |
|  |  | PEPScurrentSensor(float.A) | EPS.F_PT. F_EM, F_HM |
|  |  | SEPSvoltageSensor(float.V) | EPS.F_PT. F_EM, F_HM |
|  |  | Motor1VoltageSensor(float.V) | EPS.F_PT. F_EM, F_HM |
|  |  | Motor2VoltageSensor(float.V) | EPS.F_PT. F_EM, F_HM |
|  |  | BrakingTorqueCommand(float.(N/m)) | EPS.F_PT. F_EM |

F_CM is a gateway for communication internal to the AEH (or its software part). The other subfunctions access to the data they need by reading F_CM’s outputs.

##### Design Rational

Management of the analog and digital interfaces are performed almost independently by two different parts of the AEH ECU. Analog signals are converted into 20Hz variables and 20 Hz messages when broadcasted on the bus.

As a bus’ subscriber (receive operations) and publisher (send operations), EPS.F_PT.F_CM operates in slave mode .wt. to the bus manager hosted in MMS. Since the bus is serial there are alternate command and transfer phases. The command and transfer message formats are left undefined. Modelling of numerical exchanges is not intended to go up to signal processing at physical layer level (ISO OSI layered model).

A communication contract abstracting the concrete protocol is considered relevant for the use case and defined. It specifies:

- Topology of exchanges (sender, content, receiver),

- Some message ordering constraints,

- The timing specifications (periodic/aperiodic, frequencies, dates, max jitters etc.),

The implementation contract of a specific protocol will be addressed only if needed by the development assurance experiments.

##### Declarative part

Specifying abstractly and precisely the correctness of transfers requires a dedicated language not at hand in this first version of the use case. This section is a preliminary approach to µXAV’s communication contract.

###### Assumptions

Analog communications

- As receiver: noiselessness and accuracy of all analog values to be scanned and quantized by F_CM.

Digital communications

- The bus manager meets its specifications: conformance to the scheduling policy, causal ordering of events ensured, timeliness ensured,

- Similarly, all emitters of the network are conformant to their specification (schedule, timeliness, causal ordering).

###### Guarantees

Analog communications

- As emitter: noiselessness and accuracy of all the analog signals that are modulations of the digital values.

Digital communications

- F_CM meets its E/R slot constraints, as defined at bus management level (frames of the message scheduling policy),

- As sender F_CM is non lossy,

- F_CM ensures content integrity on all the emitted messages (transfers from ECU’s internal memory to the physical layer),

- the periodic emissions meet their rate and jitter constraints,

- the internal queuing policy of emitted event messages (e.g FBs) preserves causal ordering,

- As receiver F_CM is non lossy,

- F_CM ensures content integrity on all the received messages (transfers from the physical layer to ECU’s internal memory),

- the internal queuing policy and time stamping policy of received messages preserves causal ordering.

##### Constructive part

Protocol standard dependent (CAN bus, filed bus, AFDX, 1553 etc.).

Will be specified if handling the implementation contract is needed.

#### Health Monitoring (EPS.F_HM)

##### Interface

The input interface is designed to enable future sensor/actuator diagnosis and trend monitoring. The output interface is limited to what is computed in the first version of the use case: losses of key propulsion resources sent to F_PC which aggregates them with energy warnings to build the overall propulsion availability indicator (PropulsionOK(Boolean)).

| Inputs | From | Outputs | To |
| --- | --- | --- | --- |
| PEPSvoltageSensor(float.V) | EPS.F_PT. F_CM | FBmotor1(Boolean) | EPS.F_PT. F_PC |
| PEPScurrentSensor(float.A) | EPS.F_PT. F_CM | FBmotor2(Boolean) | EPS.F_PT. F_PC |
| SEPSvoltageSensor(float.V) | EPS.F_PT. F_CM | FBswitch(Boolean) | EPS.F_PT. F_PC |
| Motor1VoltageSensor(float.V) | EPS.F_PT. F_CM |  |  |
| Motor2VoltageSensor(float.V) | EPS.F_PT. F_CM |  |  |
| Motor1VoltageCommand(float.V) | EPS.F_PT. F_PC |  |  |
| Motor2VoltageCommand(float.V) | EPS.F_PT. F_PC |  |  |
| SwitchConfiguration({‘1-12’,’1-1’,’1-2’,’2-12’,’2-1’,’2-2’}) | EPS.F_PT. F_PC |  |  |
| pSensor(float.rad) | EPS.F_PT. F_CM |  |  |
| pdotSensor(float.(rad/s)) | EPS.F_PT. F_CM |  |  |
| qSensor(float.rad) | EPS.F_PT. F_CM |  |  |
| PEPSestimatedCapacity(Integer.%) | EPS.F_PT. F_EM |  |  |
| SEPSestimatedCapacity(Integer.%) | EPS.F_PT. F_EM |  |  |

##### Design Rational

Motors’ Failure Bits

When a motor is failing, it is supposed to deliver less torque with greater current consumption. The trend to higher energy consumption can only be detected by a small drop of tension at motors’ input ports (no current sensor at motor inlet). This threshold-based diagnostic is not robust in transients. It has to be applied only in steady state conditions. F_HM compares MotoriVoltageCommand to MotoriVoltageSensor, and monitors PEPS’ and SEPS’ capacities to filter the case where the cause of tension drop is source exhaustion instead of motor wear.

Power Switch’ (crossbar) Failure Bit

The crossbar is made of several switches that route electrical power from source i to motor p. The failure modes are non activation of a switch (electrical or electronic problem) or switch stuck in open or closed position.

They cause inlet voltage to be non conformant to the command. FB=1 in such situations.

##### Declarative part

###### Assume

- No sensor failures (unrealistic hypothesis, first version only),

- The sensor measurements are accurate (to be replaced by “within uncertainty domain” in later versions),

- The voltage commands are correct, in particular not higher than assumed by EPS design (would lead to systematic false positives),

###### Guarantee

- Motors’ failure bits are accurate or conservative: under motors’ fault model hypothesis there are no FB false negatives,

- Motors’ FB false positives are rare (< 1/1000 flight hours). They occur only in poor steady state conditions,

- The crossbar’s FB is accurate or conservative: under assumption of the sources’ and crossbar’s fault models there are no FB false negatives.

##### Constructive part

Motors’ Failure Bits

F_HM uses a tabulated function provided by the motor’s supplier to convert the voltage commands into expected voltages at motors’ inlets. F_HM uses the AV’s mechanical sensors and the braking commands to decide when flying conditions can be considered as sufficiently steady: constant speed (+- threshold) and constant altitude (+- threshold) over a 10s window. Diagnostics are repeated as many times as steady state 10s-windows occur.

Power Switch’ Failure Bit

When both motors are up, they are both powered at half power. Any measured null voltage or reference-doubled voltage is assumed to be caused by a power switch failure.

When one motor is up and the other is down, the down must be isolated from the network (F_PC is in charge). The isolated must have null inlet voltage, and the active one full inlet voltage. Any discrepancy from this configuration is a signature of crossbar failure.

When both motors are down, F_HM’s software doesn’t care about the crossbar. The two motors are isolated; Crossbar diagnostic is no longer possible. FB is set to 0, for no risk of false positive. False negatives are no longer at stake when propulsion is lost.

### HBS’ Functions

HBS is similar to EPS in spite of the different energy domain:

- Two hydraulic power sources (i.e two electrical pumps) analogous to PEPS + SEPS,

- A doubled braking clutch analogous to the doubled electrical engine,

- Spool valves to reconfigure the oil flows depending on failures, analogous to the crossbar,

- Shut-off valves to isolate the failed pumps or clutches, analogous to the isolation switches in EPS.

The sensor architecture is similar too. At source outlet level there are pressure and flow measurements, like voltage and current measurements at PEPS’ outlet. At actuator level, there is only a pressure sensor at clutch inlet, like voltage sensing at engine inlet.

See the Architecting and Implementation sections and the global model for more details.

The FDIR aspects are similar as well: fail operational resilience to all single component failures.

The main difference between EPS and HBS is the absence of joint supply by HBS’ sources. The electrical pumps, though permanently powered by EPS (save isolation cases), are operated in cold redundancy by HBS’ ECU. One pump only pressurises the unique accumulator which is shared by the two circuits that supply hydraulic power to the clutches.

Energy storage management is also different. An oil reservoir provides flow capacity while an accumulator provides pressure capacity.

Flow capacity may decrease because of possible oil leakage in pumps, pipes, valves and clutches. Some of these devices have internal leakage oil collectors that feed the leakage flows back to the reservoir. External leakage leads to oil loss. OilCapacity(float.%) gives the level of the reservoir.

There is no accumulator’s pressure measurement. Pressure sensors at pumps’ outlet and the physical characteristics of the accumulator are sufficient to estimate the hydraulic energy capacity (i.e the releasable hydraulic energy in case of fall of electrical pressurization).

#### Braking Control (HBS.F_PT.F_BC)

##### Interface

F_BC, like F_PC, essentially converts mechanical torques in the new energy domain: (pressure, flow) instead of (tension, current). It also manages the pumps’, valves’ and clutches’ failures by reconfiguring the hydraulic power lines.

| Inputs | From | Outputs | To |
| --- | --- | --- | --- |
| BrakingTorqueCommand(float.J) | HBS.F_PT. F_CM | Clutch1PressureCommand(float.Pa) | HBS.F_PT. F_CM |
| ReservoirOilCapacity(float.%) | HBS.F_PT. F_CM | Clutch2PressureCommand(float.Pa) | HBS.F_PT. F_CM |
| FBclutch1(Boolean) | HBS.F_PT. F_HM | SpoolValve1Command(float.%) | HBS.F_PT. F_CM, F_EM |
| FBclutch2(Boolean) | HBS.F_PT. F_HM | SpoolValve2Command(float.%) | HBS.F_PT. F_CM, F_EM |
| FBspoolValve1(Boolean) | HBS.F_PT. F_HM | Pump1ShutOffValveCommand(Boolean) | HBS.F_PT. F_CM |
| FBspoolValve2(Boolean) | HBS.F_PT. F_HM | Pump2ShutOffValveCommand(Boolean) | HBS.F_PT. F_CM |
| Pump1FlowSensor(float.(kg/s)) | HBS.F_PT. F_CM | Clutch1ShutOffValveCommand(Boolean) | HBS.F_PT. F_CM |
| Pump1PressureSensor(float.Pa) | HBS.F_PT. F_CM | Clutch2ShutOffValveCommand(Boolean) | HBS.F_PT. F_CM |
| Pump2FlowSensor(float.(kg/s)) | HBS.F_PT. F_CM | BrakingOK(Boolean) | HBS.F_PT. F_CM for MMS.F_PT.F_MM |
| Pump2PressureSensor(float.Pa) | HBS.F_PT. F_CM | OperatingElectricalPump({‘P1’,’P2’}) | HBS.F_PT. F_EM |
| Clutch1PressureSensor(float.Pa) | HBS.F_PT. F_CM |  |  |
| Clutch2PressureSensor(float.Pa) | HBS.F_PT. F_CM |  |  |
| pSensor(float.rad) | HBS.F_PT. F_CM |  |  |
| pdotSensor(float.(rad/s)) | HBS.F_PT. F_CM |  |  |
| qSensor(float.rad) | HBS.F_PT. F_CM |  |  |
| HydraulicCapacity(float.%) | HBS.F_PT. F_EM |  |  |

##### Design Rational

Modes

Two braking modes, like for propulsion:

- MMS Master: F_BC converts N.m requests into (Pa, kg/s) commands,

- HBS Master: when MMS’ both channels are lost. Two cases:

  - EPS is operative. The mission is being aborted by soft landing. HBS monitors the AV’s dynamics .wt. the safety envelope. When braking actions are needed to safeguard the AV, HBS delivers short and intense braking pulses to pullback the AV’s state vector towards the centre of the flight envelope,

  - EPS is no longer operative. This is detected by indirect means since HBS has no access to EPS’ sensors. If both electrical pumps deliver very low pressure, HBS considers EPS as failed. Then emergency landing is triggered (see F_EL).

Control

Contrary to propulsion which runs many control laws for performance modulation, there are only two braking laws:

- BCL_MMSmaster: pure energy conversion of MMS’ braking torque commands,

- BCL_HBSguard: HBS uses the AV’s mechanical sensors to detect when the AV dangerously nears the flight safety envelope (boarder is phase-dependent). As EPS and HBS are active independently there are no longer coordinated actuation “reversals”. HBS and EPS may have simultaneous conflicting actions on the propeller’s shaft, possibly leading to shaft breakage. The Braking Control Law computes the best moment to trigger its clear braking action (EPS’ activity must be sufficiently low and the flight safety margin still sufficient).

F_BC assumes hydraulic power is always available. F_EM is in charge of ensuring this property. F_BC acts on spool valves only, to modulate pressure releases from the accumulator.

F_BC is in charge of circuit switching and actuator control while F_EM is in charge of piloting the active electrical pump to keep maximal pressure in the accumulator. Maximal pressure means maximal hydraulic stored energy. F_EM needs to know which of the two pumps has been configured by F_BC as operating.

FDIR

There are 2 x 3 configurations of HBS’ circuits:

| Source | Actuators |
| --- | --- |
| Pump1 | Clutch1 + Clutch2 (50-50) |
| Pump1 | Clutch1 |
| Pump1 | Clutch2 |
| Pump2 | Clutch1 + Clutch2 (50-50) |
| Pump2 | Clutch1 |
| Pump2 | Clutch2 |

The shutoff valves (isolation) and the spool valves (routing + pressure modulation) are activated by the operating ECU to configure the circuits (1/6 configurations) so that the source of higher priority supplies the available actuators. Any device down must be isolated.

##### Declarative part

###### Assume

- MMS’ braking torque commands are correct,

- AV’s mechanical sensors are accurate (uncertainty model + fault model to be added in later versions),

- Hydraulic sensors are accurate (uncertainty model + fault model to be added in later versions),

- The electrical pumps are conformant to their specification (actual behaviour .wt. documentation’s response curve),

- The clutches are conformant to their specification (actual behaviour .wt. documentation’s response curve)

###### Guarantee

- Torque is conserved in the transduction from mechanical to hydraulic,

- Hydraulic commands meet the clutch’s specification (performance and safety domains TBD at model level),

- In case of single component failure, braking is kept fully operational,

- Every device diagnosed as failed (electrical pump (x2), clutch (x2)) is isolated from the hydraulic circuits by means of a suitable valve configuration at most 500ms after the diagnostic,

##### Constructive part

The design rational gives the intent of the algorithms.

The actual procedures will be available at model level, and constitute the formal system requirements allocated to the AEH or to the software part of the AEH item.

BrakingOK is set to true in two cases:

- At least one hydraulic power line is active with enough stored energy,

- Both electrical pumps are down or isolated, HBS operates on accumulator base only, but the pressure capacity is still above AccumulatorPressureHighThreshold (e.g 75%).

BrakingOK uses HydraulicCapacity(%), which in turn uses the estimated pressure level in the accumulator and the estimated oil level in the reservoir. In case of excessive external leakage, hydraulic stored energy collapses even when pressurization is nominal.

#### Energy Management (HBS.F_PT.F_EM)

##### Interface

| Inputs | From | Outputs | To |
| --- | --- | --- | --- |
| Pump1FlowSensor(float.(kg/s)) | HBS.F_PT. F_CM | Pump1Command(float.%) | HBS.F_PT. F_CM |
| Pump1PressureSensor(float.Pa) | HBS.F_PT. F_CM | Pump2Command(float.%) | HBS.F_PT. F_CM |
| Pump2FlowSensor(float.(kg/s)) | HBS.F_PT. F_CM | HydraulicCapacity(float.%) | HBS.F_PT. F_BC |
| Pump2PressureSensor(float.Pa) | HBS.F_PT. F_CM |  |  |
| OperatingElectricalPump({‘P1’,’P2’}) | HBS.F_PT. F_BC |  |  |
| SpoolValve1Command(float.%) | HBS.F_PT. F_BC |  |  |
| SpoolValve2Command(float.%) | HBS.F_PT. F_BC |  |  |
| ReservoirOilGauge(float.%) | HBS.F_PT. F_CM |  |  |

##### Design Rational

EPS powers the two pumps but is not in charge of their control, i.e modulation of the DC motors’ rotation speeds that generate modulated pressure. EPS has only authority to disconnect a pump suspected of (electrical) network instabilities.

F_EM is in charge of hydraulic energy storage in the accumulator, i.e its pressure control (see Implementation section). The two pumps can pressurize the accumulator but only one at a time. F_EM regulates pressure just below the maximum level for maximal braking capacity in case of EPS’ loss.

Over pressure of the accumulator is classified as hazardous.

And explosion as catastrophic.

F_EM uses OperatingElectricalPump to know which pump is regulating the accumulator’s pressure. It controls this pump with a [0, 1] continuous command. Conversion of the normalized command to (Pa, kg/s) is possible using the pump’s characteristic curve given by its supplier.

F_EM gets feedback of its commands by means of the pump outlet sensors. These measurements do not give the pressure inside the accumulator (which is lower, but how much lower?). Since there is no pressure sensor inside the accumulator, F_EM must make a conservative estimation of it using the spool valve commands to estimate the ensuing depressurization.

Note: once again there is no defensive design, in spite of the over pressure catastrophic failure condition. F_EM does not reconstruct its own view of which pump is active, nor does it estimate whether the spool valve commands are consistent with the AV’s dynamics. This would happen only if functional redundancy with “independence” was requested by the safety group.

##### Declarative part

###### Assume

- F_EM has an accurate estimation of pressure and oil levels at power-on time,

- F_BC’s commands are correct (operating pump, spool valves, shutoff valves),

- Suppliers’ surrogate models of response curves are correct (pump and clutch),

- Pressure and flow sensors are accurate (later: conformant to their fault model),

###### Guarantee

- Pressurization commands never lead to overpressure (hence never to explosion),

- Hydraulic capacity is accurate or conservative (if underestimated, 10% max),

##### Constructive part

Pressurization control

F_EM has an accurate initial state of the accumulator.

A Kalman filter based on a linearized model of nitrogen’s compressibility updates this state using the pump’s and spool valves’ commands, and the pressure sensors at clutch’s inlet.

Based on this state estimation, a PI controller regulates pressure at maximum value minus safety margin.

The precise equations will be given by the model.

Capacity estimation

The reservoir’s oil level is compared to a threshold to detect excessive oil leakage. When oil level is below this level, capacity is conventionally discounted to 20% which is interpreted as F_BC as “braking is under collapse because of external leakage”.

F_EM strives to keep maximal pressure over time but in strenuous braking conditions pressure may drop by 50% for a few seconds.

F_EM filters the capacity indicator from these transient drops (< ~10s). A low capacity value (e.g 60%) means a lasting pressurization problem, or a low oil level that is not yet symptomatic of an external leakage problem.

#### Communication Management (HBS.F_PT.F_CM)

##### Interface

Similarly to EPS and MMS, F_CM manages an analog interface and a digital interface.

The analog interface is made of hardwired links to sensors and hardwired links to valves and pumps. F_CM encodes/decodes from analog to digital and conversely.

The digital interface is the interface with the external world (from HBS.ECU’s standpoint).

Analog

| Inputs | From | Outputs | To |
| --- | --- | --- | --- |
| Pump1FlowSensor([0, 28V]) | ElecPump1 | SpoolValve1Command([0, 28V]) | SPvalve1 |
| Pump1PressureSensor([0, 28V]) | ElecPump1 | SpoolValve2Command([0, 28V]) | SPvalve2 |
| Pump2FlowSensor([0, 28V]) | ElecPump2 | ShutOffValve1Command([0, 28V]) | SHOvalve1 |
| Pump2PressureSensor([0, 28V]) | ElecPump2 | ShutOffValve2Command([0, 28V]) | SHOvalve2 |
| ReservoirOilGauge([0, 28V]) | Reservoir | Pump1Command([0, 28V]) | ElecPump1 |
|  |  | Pump2Command([0, 28V]) | ElecPump2 |
| Clutch1PressureSensor([0, 28V]) | Clutch1 |  |  |
| Clutch2PressureSensor([0, 28V]) | Clutch2 |  |  |
| pSensor([0, 28V]) | AV |  |  |
| pdotSensor([0, 28V]) | AV |  |  |
| qSensor([0, 28V]) | AV |  |  |

Digital

| Inputs | From | Outputs | To |
| --- | --- | --- | --- |
| BrakingTorqueCommand(float.(m.N)) | F_PT. F_CM | Pump1FlowSensor(float.(kg/s)) | F_BC, F_EM, F_HM |
| SpoolValve1Command(float.%) | F_BC | Pump1PressureSensor(float.Pa) | F_BC, F_EM, F_HM |
| SpoolValve2Command(float.%) | F_BC | Pump2FlowSensor(float.(kg/s)) | F_BC, F_EM, F_HM |
| ShutOffValve1Command(Boolean) | F_BC | Pump2PressureSensor(float.Pa) | F_BC, F_EM, F_HM |
| ShutOffValve2Command(Boolean) | F_BC | ReservoirOilGauge(float.%) | F_EM, F_HM |
| Pump1Command(float.%) | F_EM | Clutch1PressureSensor(float.Pa) | F_EM, F_HM |
| Pump2Command(float.%) | F_EM | Clutch2PressureSensor(float.Pa) | F_EM, F_HM |
|  |  | pSensor(float.rad) | F_BC, F_EL, F_HM |
|  |  | pdotSensor(float.(rad/s)) | F_BC, F_EL, F_HM |
|  |  | qSensor(float.rad) | F_BC, F_EL, F_HM |

#### Health Monitoring (HBS.F_HM)

##### Interface

In this version of the use case the pumps, clutches and valves can fail. Pipes and sensors are failure free (unrealistic for sensors). F_HM builds FBs for all the failure prone equipments.

The FBs are sent internally to F_BC and F_EM, and externally to MMS.F_HM via HBS.F_PT.F_CM and MMS.F_PT.F_CM, for gathering with other FBs and display on CP’s LEDs.

| Inputs | From | Outputs | To |
| --- | --- | --- | --- |
| Pump1FlowSensor(float.(kg/s)) | F_CM | Pump1FB(Boolean) | F_BC, F_EM, F_CM |
| Pump1PressureSensor(float.Pa) | F_CM | Pump2FB(Boolean) | F_BC, F_EM, F_CM |
| Pump2FlowSensor(float.(kg/s)) | F_CM | Clutch1FB(Boolean) | F_BC, F_EM, F_CM |
| Pump2PressureSensor(float.Pa) | F_CM | Clutch2FB(Boolean) | F_BC, F_EM, F_CM |
| pSensor(float.rad) | F_CM | SpoolValve1FB(Boolean) | F_BC |
| pdotSensor(float.(rad/s)) | F_CM | SpoolValve2FB(Boolean) | F_BC |
| qSensor(float.rad) | F_CM |  |  |

##### Design Rational

Pump Diagnostic

By comparing the expected effect to the measured effect at pump outlet. Two tabulated command-response curves with uncertainty margins are used. They are given by the pump’s supplier, a surrogate model: (input = commanded %) => (output = (pressure, flow)).

Spool Valve Diagnostic

By comparing the spool valves commands to the pressure at clutch’s inlet. Same principle and means as for pumps.

Clutch diagnostic

The pressure sensed at clutch inlet is an indication of upstream activity; alone it can’t help diagnosing the clutches. One needs the AV’s mechanical response to assess clutches’ efficiency. But in braking phases, AV’s mechanical response does not exclusively depend on clutches’ action. Depending on situations it may also depend on wind, and even on propulsion if MMS is down. Clutch diagnostic is reliable only when the AV’s state vector gives information on “pure” clutch actuation. F_HM has to make a balanced decision between:

- being over pessimistic on clutch’s lack of efficiency, but conservative .wt. safety,

- being over optimistic on clutch’s efficiency, which may be favourable to the mission success ratio. But to the failure ratio alike...

##### Declarative part

###### Assume

- All sensors are accurate or conformant to their uncertainty specification.

###### Guarantee

- No false negatives (no non detected actual failures). In-flight diagnosis is essential for safety, and on ground for dispatch.

#### Emergency Landing (HBS.F_EL)

Admittedly, braking-only control of a drone sounds strange. What is so-called “hard landing” if not some sort of stone fall control?!

As already mentioned, to limit the size of the use case no true flight mechanics was introduced. But to support physically and functionally consistent development and development assurance experiments a true controlled mechanical dynamics was introduced. Because of this mechanical intended interpretation, on purpose not explained, an actual implementation of the “µXAV product” could be possible.

In this intended interpretation the so-called ‘braking-only’ control of the drone’s descent makes sense.

##### Interface

The interface is proposed as digital. The control law is envisioned as being programmed on a FPGA-based ECU. A pure analog hardware control could be envisioned as well. The interface is prescriptive as far as functional exchanges are concerned. The implementation technology is left open.

| Inputs | From | Outputs | To |
| --- | --- | --- | --- |
| pSensor(float.rad) | F_CM | Clutch1PressureCommand(float.Pa) | F_CM |
| pdotSensor(float.(rad/s)) | F_CM | Clutch2PressureCommand(float.Pa) | F_CM |
| qSensor(float.rad) | F_CM | SpoolValve1Command(float.%) | F_CM |
| HydraulicCapacity(float.%) | F_EM | SpoolValve2Command(float.%) | F_CM |
| Pump1PressureSensor(float.Pa) | F_CM |  |  |
| Pump2PressureSensor(float.Pa) | F_CM |  |  |
| StartEmergencyLanding(Boolean) | MMS. F_PT. F_MM |  |  |

##### Design Rational

Since F_EL can be triggered at any moment (safety escapes) by MMS or HBS, and must be handled most responsively, the three systems update the AV’s state vector (altitude, speed, attitude) in hot redundancy (no state transfer at hand over).

When MMS and EPS are operative, MMS.F_MM is in charge of triggering the emergency landing because it holds more mission and energy related data than HBS to make such a critical and irreversible decision.

If MMS or EPS is down, HBS decides alone, on the sole basis of the (p, pdot, q, qdot) state vector (.wt. the safety envelope).

In both triggering cases emergency landing is handled by HBS alone (braking only control).

First, F_EL assesses the energetic situation: will it be accumulator only or is EPS operative? In which later case a “softer hard landing” can be planned by the F_EL controller.

Think “softer hard landing” as a glide where time is left to target the landing place (time scale = k.minutes).

If EPS is down, only a fast “best effort” approach is tractable: optimal use of the accumulator’s capacity for fastest descent (time scale =  k.seconds). Then, two cases after the best effort phase:

- hard landing: enough hydraulic energy was left in the accumulator to decay nearly all the AV’s mechanical energy when altitude 0 is reached, with speed being as low as possible for limited ground impact,

- crash: altitude is not 0 when the accumulator runs empty... Some mechanical energy (kinetic + potential) remains, the ground will be in charge of decaying it.

##### Declarative part

###### Assume

- AV’s state vector is accurate,

- Hydraulic capacity is accurate or conservative (at most 10% under approximation).

###### Guarantee

- F_EL’s internal activation logic is correct:

  - No false positive: EPS or MMS is down and µXAV escaped the safety envelope when activation=true,

  - No false negative: one of the above conditions is falsified if activation=false.

- F_EL is activated when externally requested by MMS (StartEmergencyLanding message),

- In AWC or SWC, when EPS is operative, F_EL always succeeds in landing the drone at speed < MaxLandingSpeed (e.g 40kt).

##### Constructive part

Optimal control algorithm with a joint hydraulic energy and AV’s speed criterion.

To be detailed at model level.

### MMS’ Functions

#### Mission Management (MMS.F_PT.F_MM)

This function is a direct mapping of the DIF described at AV, with some adaptation of the interface (e.g GS data now is available from MMS.F_PT.F_CM).

MMS.F_PT.F_MM ~ AV.F_PT.F_MM.

#### Energy Management (MMS.F_PT.F_EM)

##### Interface

| Inputs | From | Outputs | To |
| --- | --- | --- | --- |
| ConsumedPEPSenergy(float.J) | MMS.F_PT. F_CM | EnergieOK(Boolean) | MMS.F_PT. F_MM |
| ConsumedSEPSenergy(float.J) | MMS.F_PT. F_CM |  |  |
| ConsumedMechanicalEnergy(float.J) | MMS.F_PT. F_MM |  |  |
| ConsumedHydraulicEnergy(float.J) | MMS.F_PT. F_CM |  |  |
| MechanicalEnergyBudget(float.J) | MMS.F_PT. F_MM |  |  |

##### Design Rational

First, there are energy losses everywhere (mechanical frictions in propulsion/breaking, electrical dissipation in the circuits, fluid heating etc.), and nobody knows to what extent. So what is consumed to control the drone is significantly greater than not what is sensed as effect on it.

Second, estimation of PEPS’ and SEPS are imprecise. Battery load estimation is an issue. In particular it is sensitive to temperature. To face these uncertainties while being conservative for safety may lead to over pessimistic mission management.

Third, randomness of weather conditions for the rest of the mission. Will the future be analogous to the past 10 mn? No weather forecast in the drone, even in GS-connected mode (but envisioned for upgrades). Hydraulic energy consumption is a way to estimate if there are turbulent or steady wind influences.

MMS.F_PT.F_EM has to analyse these 4 consumption estimates, compare them to the capacities at take-off (assumed known constants), compare to the budget notice by F_MM, and make a decision: is energetic feasibility’s likelihood high enough? YES/NO = the returned Boolean.

##### Declarative part

###### Assumptions

- Energy consumption estimates are accurate or conservative (with upper bound at 95% CL TBDL),

- Budget notice is accurate or conservative (with upper bound at 95% CL TBDL).

###### Guarantees

- Soundness of the decision:

  - OK=true. If the future meets the feasibility hypotheses used by the algorithm (TBDL), then the mission runs to completion (under assumption of no breakdown).

  - OK=false. If the future is conformant to AWC, mission failure is certain.

##### Constructive part

First version with a deterministic algorithm: ((capacity1 + capacity2) - (budget + risk_margin(disturbance_indicator)) > 0).

Second version with an appropriate statistical decision algorithm in order to meet, in a way accountable to Authorities:

- the success ratio objectives,

- the catastrophic failure rate (case of OK until unnoticed exhaustion),

#### Communication Management (MMS.F_PT.F_CM)

Because MMS’ ECUs have analog access to the mechanical sensors and to the CP’s MMI devices, two distinct interfaces are to be considered.

##### Analog Interface

| Inputs | From | Outputs | To |
| --- | --- | --- | --- |
| pSensor [0,28V] continuous signal encoding [-π,+π] | AV | READY_indicator [0,28V] discrete signal | CP |
| pdotSensor [0,28V] continuous signal encoding [-π20/s,+20π/s] | AV | CANCELLED_indicator [0,28V] discrete signal | CP |
| qSensor [0,28V] continuous signal encoding [-π,+π] | AV | COMPLETED_indicator [0,28V] discrete signal | CP |
| ON/OFF [0,28V] discrete signal | CP | ABORTED_indicator [0,28V] discrete signal | CP |
| START [0,28V] discrete signal | CP | CP_LEDs(bitarray[20]) 20x [0,28V] discrete signals | CP |
| A/RP [0,28V] discrete signal | CP | PEPSindicator [0,28V] continuous signal encoding [1,100] | CP |
| OPEN/CLOSED [0,28V] discrete signal | CP | SEPSindicator [0,28V] continuous signal encoding [1,100] | CP |
| Rotactor1 [0,28V] continuous signal encoding [0,9] | CP |  |  |
| Rotactor2 [0,28V] continuous signal encoding [0,9] | CP |  |  |

MMS codes a copy of ON/OFF for state awareness only, especially in the initialization phase. ON/OFF is an electrical switch belonging to EPS.

Present CP is a bit bizarre as numerical connection is needed for the USB port but the indicators are analog devices. The indicators could become digital in later versions.

##### Digital Interface

As the two ECUs exchange their respective mechanical sensor values and navigation state, ECUi, ECUj notations are used to denote the ECU1 ↔ ECU2 exchanges. The table is assumed to be ECUi’s.

| Inputs | From | Outputs | To |
| --- | --- | --- | --- |
| USB_port (TBC) | CP |  |  |
| HandOverECUjToECUi | MMS.ECUj | HandOverECUiToECUj | MMS.ECUj |
| ConsumedMechanicalEnergy(float.J) | MMS.ECUj | ConsumedMechanicalEnergy(float.J) | MMS.ECUj |
| MechanicalEnergyBudget(float.J) | MMS.ECUj | MechanicalEnergyBudget(float.J) | MMS.ECUj |
| HealthMonitoringState([bitarray]). (TBC) | MMS.ECUj | HealthMonitoringState([bitarray]). (TBC) | MMS.ECUj |
| ConsumedPEPSenergy(float.J) | EPS.ECUi,j | PropulsionTorque(float.(m.N)) | EPS.ECUi,j |
| ConsumedSEPSenergy(float.J) | EPS.ECUi,j | BrakingTorque(float.(m.N)) | EPS.ECUi,j |
| ConsumedHydraulicEnergy(float.J) | HBS.ECUi,j | pSensor(float.rad) | MMS.F_PT.F_CM |
| EPSHealthMonitoringState([bitarray]). | EPS.ECUi,j | pdotSensor(float.(rad/s)) | MMS.F_PT.F_CM |
| HBSHealthMonitoringState([bitarray]). | HBS.ECUi,j | qSensor(float.rad) | MMS.F_PT.F_CM |

##### Design Rational

The fault tolerance rational is the main design driver of communications:

- Master-slave design pattern: MMS is the master, EPS and HBS are the slaves. Authority concerns AV’s control, mission management and emergency landing decision. MMS only manages a mission. EPS and HBS can take over after MMS failure, but only for soft or hard landing. So communications have a “star” pattern, with MMS at the centre and two branches (EPS, HBS).

- There is hot redundancy only for ‘fast’ dynamics ([10Hz,20Hz]), i.e AV’s control in AWC. This is true for MMS, EPS, HBS. In the three systems the twin ECUs do not exchange data on sensors or state vectors. Both twins get all sensor information and perform all control computation. There is only one HandOver message when the leader transition from ECU1 to ECU2.

- Cold redundancy: energy management and health monitoring (slow dynamics). When transitioning from ECU1 leader to ECU2 leader, ECU1 sends its state vectors related to energy levels, energy budgets, and component failures to ECU2.

- In case of MMS failure CP is no longer managed.

The intent of the use case is to leave the communication protocol underspecified, so that certification be as robust as possible .wt. to change in message formats and scheduling.

However communications plays a crucial role in distributed control and AV’s safety, so a communication contract has to specify to what extent protocol abstraction can be made.

For modelling and “completeness” reasons a static scheduling policy is given.

##### Declarative part

###### Assumptions

None.

###### Guarantees

- No message loss,

- Content integrity is preserved,

- Causal ordering is preserved within EPS, HBS, MMS,

- Timeliness (deadlines on periodic and aperiodic messages + jitter on fast dynamic messages) is ensured,

##### Constructive part

Figure 15 proposes a static scheduling policy when MMS.ECU1 is bus manager. The same policy is applied when MMS.ECU2 takes over because of ECU1 failure.

Figure 15: Broadcast scheduling policy

The bus manager:

- polls the GS, and gets mission preparation or mission updates (1),

- sends the drone “mission state vector” to the GS operator (2),

- schedules download information to its ‘slaves’ (3):

  - Torque commands to EPS’ and HBS’ twins (10Hz)

  - HandOver message to its own twin (10Hz)

  - Energy capacity + budget, health monitioring state to its twin (1Hz)

- Upload information from EPS (4)

  - HandOver message from EPS.ECU1 to EPS.ECU2

  - EPS.ECU1’s state for cold redundancy start-up of EPS.ECU2

  - PEPS and SEPS updated capacities

  - Failure bit array

- Upload information from HBS (5)

  - HandOver message from HBS.ECU1 to HBS.ECU2

  - HBS.ECU1’s state for cold redundancy start-up of HBS.ECU2

  - Update of hydraulic energy capacity

  - Failure bit array

#### Health Monitoring (MMS.F_HM)

At present it only collects its own BITEs, the EPS’ and HBS’ FB arrays and sends the health monitoring bundle to MS

### MS’ Functions

Presently MS supports a unique diagnostic function (MS.F_HM). Since MS shares hardware with MMS it has no proper communication function. In other words:

MS.F_CM = MMS.F_CM

All ECUs have self-diagnostics. They send their BITE on the bus to MMS which in turn publishes them to MS.

Sensors and actuators have no diagnostic capabilities.

Their diagnostic is indirect, by detecting behavioural-informational inconsistencies. These indirect diagnostics are performed per physical domain (mechanical, electrical, hydraulic). MMS.F_HM is in charge of detecting abnormalities in the mechanical sensors. The mechanical actuators are indeed two electrical motors handled by EPS and two hydraulic clutches handled by HBS.

#### Interface

| Inputs | From | Outputs | To |
| --- | --- | --- | --- |
| BITEs of EPS’ and HBS’ ECUs | MMS.F_CM | CP_LEDs(bitarray[20]) | MMS.F_CM for CP’s LEDs |
| BITEs of MMS’ ECUs | MMS.F_CM MMS.F_HM |  |  |
| Other Failure Bits (TBDL) | EPS.F_HM, HBS.F_HM MMS.F_HM |  |  |

#### Declarative part

Assumptions: none.

Guarantees: all LED bits are associated to a system part named by a label stuck the CP’s front end. All BITE bits copied in the LED bits originate form system items. Labels, copies and origins are consistent. (TBDL by a consistency table).

#### Constructive part

At present the function only copies the elaborated FBs into the appropriate LED bits.

# IMPLEMENTATION

The case study needs to be precise on what the final µXAV product is, especially for working on OP Correctness and its constraints related to tiers.

In this first version no actual drone will be used at implementation stage. Using a low cost commercial kit (quadri or hexacopter) may be considered for future versions.

The environment models of EPS, HBS, AV’s airframe and communication network, are used for executable system specifications (model-based system engineering), and can be reused for software validation and system validation with “simulation in the loop”.

In this section some orientations are given on the implementation stage. They are still preliminary and need to be supplemented by RESSAC WG.

Since on the physical aspects, especially HBS, EPS, and AV’s body, the “final product” is only a model, the physical devices and the energy levels were chosen greater than that of a small off-the-shelf quadricopter or hexacopter. This part of the use case would have to be entirely reworked if a “true final product” option were chosen in the future.

This first version is expected to be suitable for OP-based experiments on processes that make extensive use of environment virtualization at integration/validation stage.

Specifications of the four following sections (EPS’, HBS’, MMS’, MS’) could be found in the “Architecting” section. But the “Architecting” section was made to contain only structural information (nodes, connections, dependencies, redundancies, etc.), not the technological attributes characterizing the nodes and links of the architecture. It is a safety process oriented section (e.g CCA, CMA).

The OPs do not refer to the architecture, even in the prerequisites or constraints on the arguments. The architecture is contained in the implementation (a pre-requisite), but is it exclusively contained in the implementation? If implementation is purely constructive (i.e property-less), where are stated the architectural properties that are structural, not behavioural? Is DIF exclusively functional, i.e behavioural?

## Electrical Propulsion System

EPS has a dual source dual controller architecture to meet the safety and availability requirements. The controllers are custom SoCs developed in conformance to DO-254 (see SAFRAN’s document).

Some technological characteristics are the following:

- The design is High Voltage Continuous Current (HVCC): the primary and the secondary sources deliver 270V tension,

- The two propulsion engines are DC motors,

- The type of the primary source is not defined yet (candidate solutions are super capacitors, Li-Ion batteries or small hydrogen fuel cells). The primary has the greatest energy capacity, but may be short of power on worst case load profiles,

- The secondary source is a Li-Ion battery. Its energy and power densities are respectively ~150Wh/kg and ~400W/kg. Its mass is a design parameter.

- The secondary is continuously reloaded by the primary to ensure full capacity when power demand peaks in emergency situations,

- In normal mode both sources are connected to the two motors:

  - for fault tolerance (no single component failures),

  - for power balancing: the secondary source is requested to supply additional power in demanding transients (hazardous wind gusts),

- In some failure cases the primary source has to be isolated from the circuits by the controller (source drain avoidance),

- The two sources deliver electrical power to HBS’ electrical pumps (power of the pumps’ DC motor is a design parameter),

- As mentioned in the EPS’ DIF, the AEH controller is in charge of:

  - Digital communications on the bus,

  - AC/DC conversion of EPS’ analog links (3 mechanical sensors),

  - tension commands for the propulsion motors (voltage control of the circuits, 10Hz),

  - command of the switches (FDIR configurations),

  - monitoring of the sensed tensions and currents,

  - Energy consumption estimation, remaining capacity estimation (primary and secondary),

  - Local diagnostic-prognostic analysis (EPS.F_HM)

## Hydraulic Breaking System

HBS has a dual architecture, with some form of 3-redundancy at source level. Some technological indications on the components used in HBS:

- The hydraulic fluid is mineral oil with freezing point lower than -80°C and ignition point higher than +150°C. µXAV has no air conditioning system. The equipment bay is small, overheat (excessive pressure or fluid dynamics) and freezing (operations in low/high latitudes) are design issues. No oil contamination aspects in the use case.

- A reservoir (with internal leakage backflow) supplies fluid to the pumps and compensate for volume fluctuations (thermal expansion/contraction, actuation modulation),

- There are 3 pressure sources (~ (1000, 2000] psi):

  - 2 identical electrical gear pumps (DC motors), powered by EPS (4 configurations: PEPS-PEPS, PEPS-SEPS, SEPS-PEPS, SEPS-SEPS),

  - An accumulator (nitrogen compression). The accumulator has a double function: it dampens excessive pressure or flow fluctuations (e.g switch of shut-off valves) and it releases hydraulic power in case of double pump failure (F_EL only).

- A pressure sensor and a flow sensor at pump outlet,

- A pressure sensor is at clutch inlets (speed braking actuators),

- No valve sensors,

- Similarly to propulsion, speed braking is performed by double actuation, each with its own hydraulic line. Because of 10Hz dynamics inductance and capacitance of the lines have to be considered (hydraulic stability),

- Digitally controlled spool valves reconfigure the oil paths depending on HBS’ component failures (fail operational, fail safe policies),

- Digitally controlled servo valves modulate pressure/flow in the two lines (torque commands),

Dimensioning of HBS’ components will be addressed in the following order:

- Mission performances constrain the electrical engines, brake torques, primary and secondary electrical sources.

- Once the feasibility space of 1) is solved, the pump type and the accumulator type will be chosen to meet the worst case demands.

- The software control laws’ parameters (at HBS level and MMS level) will have to be tuned to fit the physics (reworked after any physical device modification).

## Mission Management System

MMS is a dual channel real-time computer running a mono-task mono-rate non interruptible software. Real-time multi-task scheduling may be considered in later versions.

The two instances of the software are (quasi)-identical. They run in semi-active redundancy. Redundancy policy:

- Channel 1 is master when both channels are operative,

- Cold redundancy: communication with GS, MS’ functions, mission supervision (range monitoring only),

- Hot redundancy: mechanical control (so-called FCS part of mission supervision) in MMS, electrical and hydraulic control in dual ECUs of EPS and HBS.

### Software Implementation

This section is limited to MMS’ software. The software components of the AEH items (SoC and FPGA) are discussed in the next section.

Four implementations of the MMS’ DIFs:

- C code generation from the MMS’ software model of the Modelica multi-system model,

- SCADE implementation derived from a SysML model (SCADE System + KCG),

- Contract-based manual coding in ADA with Spark Ada 2015,

- Contract-based manual coding in C with Frama-C,

### Hardware Implementation

TBD by SAFRAN. SoCs and FPGAs will include software aspects.

## Maintenance System

In this first version the maintenance system is limited to timed Boolean diagnostics. MS is a provision for the later versions with a trend monitoring function (wearing of pumps, DC motors, sensors) and prognosis (MTTR).

Development assurance on statistical algorithms (e.g state estimation, machine learning) suitable for credits on maintenance or dispatch, will be addressed.

# Appendix A : Overarching PROPERTIES

Overarching Property Statement:

Intent: The defined intended functions are correct and complete with respect to the desired system behavior.

Definitions: words / phrases in the Overarching Property statement

- Desired system behavior: System needs and constraints expressed by the stakeholders.

- Defined intended functions: The record of the system needs and constraints as expressed by stakeholders.

- Failure Condition(s):  A condition having an effect on the aircraft and/or its occupants, either direct or consequential, which is caused or contributed to by one or more failures or errors, considering flight phase and relevant adverse operational or environmental conditions or external events.  (From ARP 4754A)

Pre-requisites: which must exist to allow Overarching Property satisfaction to be shown

- Defined intended functions exists.

- Failure conditions are defined for the aircraft systems.

DALs are assigned using the failure condition classifications.

Constraints: on how Overarching Property satisfaction may be demonstrated

a.	The process to satisfy this Overarching Property must be defined and conducted as defined.

b.	The defined intended functions must address the failure conditions.

c.	Criteria for evaluating the artifacts are defined and shown to be satisfied individually and collectively.

d.	All artifacts required to establish the Overarching Property are under configuration management and change control.

Assumptions: which need only be stated, not justified (if any)

- Stakeholders have the system knowledge to express the desired system behavior.

- Performing system safety assessments is not covered by these Overarching Properties.

Overarching Property Statement:

Correctness: The implementation is correct with respect to its defined intended functions, under foreseeable operating conditions.

Definitions: words / phrases in the Overarching Property statement

- Implementation: Item or collection of items contributing to system realization, for which acceptance or approval is being sought; item (from ARP 4754A) is a hardware or software element having bounded and well defined interfaces.

- Defined intended functions: The record of the system needs and constraints as expressed by stakeholders.

- Foreseeable operating conditions: External and internal conditions in which the system is used, encompassing all known normal and abnormal conditions.

Pre-requisites: which must exist to allow Overarching Property satisfaction to be shown

- Defined intended functions exists.

- The implementation of the functions exists.

- The record of the foreseeable operating conditions exists.

Constraints: on how Overarching Property satisfaction may be demonstrated

- The process to satisfy this Overarching Property must be defined and conducted as defined.

- When tiers of decomposition are used, the means of showing correctness among the tiers and to the defined intended functions must be defined and conducted as defined.

- The implementation must be correct when functioning as part of the integrated system or in environment(s) representative of the integrated system.

- Criteria for evaluating the artifacts are defined and shown to be satisfied individually and collectively.

- All artifacts required to establish the Overarching Property are under configuration management and change control.

- All design and manufacturing data to support consistent replication of the type design and instructions for continued airworthiness must be established.

Assumptions: which need only be stated, not justified (if any)

None.

Overarching Property Statement:

Necessity: All of the implementation is either required by the defined intended functions or is without unacceptable safety impact.

Definitions: words / phrases in the Overarching Property statement

- Unacceptable Safety Impact: The impact effect does compromise the system safety assessment.

- Defined intended functions: The record of the system needs and constraints as expressed by stakeholders.

- Implementation: Item or collection of items contributing to system realization, for which acceptance or approval is being sought; item (from ARP 4754A) is a hardware or software element having bounded and well defined interfaces.

Pre-requisites: which must exist to allow Overarching Property satisfaction to be shown

- Defined intended functions exists.

- The implementation or a representation of the implementation exists.

- The system safety assessment exists.

Constraints: on how Overarching Property satisfaction may be demonstrated

- The process to satisfy this Overarching Property must be defined and conducted as defined.

- The system safety assessment must address all of the implementation.

- Criteria for evaluating the artifacts are defined and shown to be satisfied individually and collectively.

- All artifacts required to establish the Overarching Property are under configuration management and change control.

Assumptions: which need only be stated, not justified (if any)

- For a TSOA appliance there may not be a complete system safety assessment for the final installation at the appliance level.

Development here includes verification and validation